# VMware Pro Cloud Services for Private Cloud - VMware Cloud Foundation 9.x Low-Level Design and Architecture

You can find the most up-to-date technical documentation on the Broadcom website at:

https://techdocs.broadcom.com/

Broadcom®

3421 Hillview Ave.

Palo Alto, CA 94304

www.vmware.com

Copyright© 2026 Broadcom® All rights reserved.

Contents

1.	Version History	4

2.	Document Overview	5

Executive Summary	5

Document Focus	5

Physical Design	5

Sizing Workbook	6

Naming Convention Document	6

Solution Specification Workbook(s)	6

Solution Verification Workbook(s)	6

3.	VMware Pro Cloud Service for Private Cloud Infrastructure Design	8

VMware Cloud Foundation Detailed Design	8

VCF Fleet in a Single Site with Minimal Footprint	8

Planning and Preparation for VCF Fleet in a Single Site with Minimal Footprint	12

VCF Fleet Design	14

Physical Infrastructure Design	17

Virtual Infrastructure Design	19

VCF Operations Design	78

Information Security and VCF Identity Broker Design	90

4.	References	105

# Version History

| Date | Ver. | Author | Contributors | Description | Reviewers |
| --- | --- | --- | --- | --- | --- |

Important	Amend the text as needed and address the consultant and or architect notes. After completion, the notes can be removed. This text provides either sample guidance information, configuration recommendations for specific services, or customer-specific configurations.

# Document Overview

This solution design document is created by VCF Professional Services to support the deployment of the Solution for Customer.

This document may be accompanied by other documents that focus on the implementation and operation aspects of the engagement, such as installation guides, solution specification, and verification workbooks if implementation services are being performed.

## Executive Summary

This executive summary aims to provide a short and comprehensive overview of the technical design, making it easier for decision-makers to understand the project's importance, benefits, and requirements without diving into the technical details.

Important	Fill out this with the high-level business language in the below for the customer environment.

This design document outlines the plan for Customer <purpose of the engagement> by VMware by Broadcom Professional Services. The aim is to improve scalability, operational efficiency, and cost-effectiveness while ensuring a high level of security and compliance. The scope of this engagement includes <list main scope items>.

Developed using the VMware by Broadcom design methodology, the architecture aims to address <customers main problems and issues>. Some of the key features of the proposed design includes <technical capabilities that will be an output of the design>.

Upon approval, the next steps include extending the design to the implementation teams, initiating the integrations, and providing the teams with the necessary operational support as part of the handover.

## Document Focus

This document, the Low-Level Design Document, is one of five main documents which collectively form the Low-Level design documents for the solution.

This includes the following content, which may be presented as one or more technical documents as required by the solution.

### Physical Design

The physical design serves as the final, implementable blueprint that translates the high-level objectives and architectural outlines of the conceptual and logical designs into specific, actionable details. The physical design details the exact hardware, including server models, storage arrays, and network devices, as well as their configurations.

It provides intricate network topologies, complete with IP schemes and VLAN settings, specifies storage layouts down to RAID levels and disk types, and outlines concrete security mechanisms such as firewall configurations and encryption protocols. The physical design also typically covers specifics on availability and disaster recovery solutions such as failover clusters and backup schedules as appropriate for customer requirements.

The physical design incorporates the management and monitoring tools to be used and how they will be configured. Essentially, the physical design is the comprehensive guide that the implementation team will follow to build the system, ensuring that it meets both functional and non-functional requirements and aligns closely with the organizational objectives.

### Sizing Workbook

The sizing workbook is a vital tool that systematically outlines the resource requirements such as CPU, memory, storage, and networking that are needed for a specific architecture.

The sizing workbook is a living document which incorporates various elements, including detailed calculations guided by formulas to ensure that the architecture can meet the calculated sizing which ideally would align with the organization's objectives and constraints.

The sizing document incorporates the licensing which will be consumed by the solution and if any design decision violates the licensing constraints or if additional licenses need to be procured part the scalability exercise.

### Naming Convention Document

The naming convention document serves as a critical blueprint for organizing and managing elements within a cloud infrastructure. This document delineates a standardized naming scheme, covering elements of the VMware by Broadcom solution to ensure uniformity, easier management, and streamlined troubleshooting.

The naming convention outlines specific syntax rules including permitted characters, name length, and capitalization guidelines, as well as semantic conventions to imbue names with functional, environmental, or locational significance.

Additionally, the document addresses rules for prefixes or suffixes. The <component> naming convention document is considered a structured approach to achieve communication among team members and stakeholders, and it aids significantly in the complex task of managing the solution.

### Solution Specification Workbook(s)

The solution specification workbook(s) serve as a set of guides that outline the specific settings required to implement a VMware by Broadcom solution. These detailed documents the precise configurations for each solution, aligning closely with the earlier conceptual, logical, and physical design phases.

Solution specification workbooks cover a wide range of elements, from exact hardware settings for servers, storage, and network devices to software versions and patches, as well as configurations like VM sizes and resource allocation. The workbooks also incorporate network settings, including VLANs and IP schemes, layout storage configurations, and detail mechanisms for high availability, disaster recovery, and security.

The solution specification workbooks ensures that deployment consultants and cloud administrators have a consistent, accurate roadmap for successfully realizing the architectural design and meeting business objectives.

### Solution Verification Workbook(s)

The solution verification workbook(s) act as structured guides for the comprehensive testing and validation of the VMware by Broadcom solution prescribed within the architecture. These workbooks are pivotal for quality assurance, defining test objectives aligned with the solution's functional and non-functional requirements, outlining specific test scenarios and procedures.

The solution verification workbooks provide detailed step-by-step instructions for executing each test, including the setup requirements, and expected outcomes, and feature explicit pass/fail criteria to quantify success or failure. The workbooks also have designated sections for recording actual test results, documenting anomalies or issues encountered.

The solution verification workbooks are an anchor for ensuring the implemented VMware solution is ready for production, meeting both technical specifications and organizational goals.

# VMware Pro Cloud Service for Private Cloud Infrastructure Design

This section describes the product design that is used for Customer for the Private Cloud Infrastructure.

VMware Cloud Foundation is a prescriptive design construct that has a fixed design for many design considerations. This document describes the design as created for the VMware Pro Cloud Services for Private Cloud Infrastructure implementation. The design is based off the VMware Cloud Foundation design with modifications to support the service.

This design also uses the VMware Validated Solution for Identity and Access Management to help secure the environment.

## VMware Cloud Foundation Detailed Design

This section describes the detailed design for the VMware Cloud Foundation deployment.

This includes the detailed design for the following components:

- ESXi Hosts

- vSAN

- vCenter Server and supplemental features

- NSX and supplemental features

- SDDC Manager

- VCF Operations

- VCF Operations Fleet Manager

See the VMware Pro Cloud Service for Private Cloud Infrastructure High Level design document, for further details on the conceptual and logical architectures and rack layout for this design.

## VCF Fleet in a Single Site with Minimal Footprint

VCF fleet design in a single site with minimal footprint is a design blueprint where the management domain of the first VCF Instance is deployed with a minimal footprint and where management components and workloads are co-located.

This Design Blueprint can be used as a full end-to-end design for a VMware Cloud Foundation platform or as a starting point and adjusted to suit your specific objectives by substituting any of the design selections listed below with alternative models.

To meet the design profile, all management and workload components are deployed in a single vSphere cluster. To support VCF Automation with the All Apps Organization model, a second cluster is required.

Figure 3-1. VCF Fleet in a Single Site with Minimal Footprint

Design Profile

This blueprint is well suited to customers who are looking for the following design profile for their VMware Cloud Foundation platform.

| Attribute | Value |
| --- | --- |
| Consumption | - Self-service consumption via VCF Automation ** - vSphere Supervisor - Direct vCenter consumption |
| Physical Site Configuration | Single Site (Single instance) |
| Availability | - Tolerates failure of a single ESX host - Tolerates failure of an individual network path |
| Isolation | - Hypervisor-based (logical isolation) - Tenant / Tenant isolation |
| Recoverability | This blueprint is not well suited for adding a disaster recovery solution. This blueprint can leverage a backup and restore functionality to provide a recovery option to the same site, or another site with suitable planning and preparation. |
| Expansion | For available resource and availability expansions, see Expansion after Design Blueprint Deployment. |

Tip	See Design Blueprints for VMware Cloud Foundation section for information on site configurations, disaster recovery and expansion.

Management Domain Design Selections

The following models for the management domain align with the design profile for this blueprint.

| Layer | Selected Model | Additional Information |
| --- | --- | --- |
| Consumption | Simple VCF Automation Model** | Single VCF Automation node that uses vSphere HA as an availability mechanism. |
|  | Fleet Level Components on Shared Management Network Model | Fleet Level management components are deployed to the same vCenter distributed port group used for the VCF instance management components. |
|  | Single Management Zone with Combined Workload Zones Model** | Single vSphere Zone used for vSphere Supervisor management and application workloads. |
|  |  | The model uses a primary (Tier-0) gateway configured in active/standby mode. It provides a VPC-compatible workload domain with network connectivity established via a centralized transit gateway. Alternative models conforming to the Blueprint profile: - NSX Segment Network Virtualization Scalable Workload Networking Model - VLAN backed Port Groups |
| Operations | Simple VCF Operations Model | Single node of all the VCF Operations components that use vSphere HA as an availability mechanism. |
|  | Fleet Level Components on Shared Management Network Model | The fleet level management components can be deployed to the same vSphere Distributed Switch port group used for the VCF Instance management components, this can be achieved using VCF Installer during initial deployment of the VCF fleet. |
| Security Governance and Compliance | Embedded VCF Identity Broker Model | VCF Identity Broker runs as a service within the management domain vCenter. |
|  | VCF Fleet-Wide Single Sign-On Model | A single VCF Identity Broker services all VCF Instances in your VCF Fleet. Alternative models conforming to the Blueprint Profile: - Cross VCF Instance Single Sign-On Model - Single VCF Instance Single Sign-On Model |
| Virtual Infrastructure | Management Domain Model | First workload domain deployed in a VCF Instance. |
|  | vSphere Single-Rack Cluster Model | vSphere cluster with all nodes in the same physical rack. Alternative models conforming to the Blueprint Profile: - Layer 2 Multi-Rack Cluster Model |
|  | Single Distributed Switch Model | Single vSphere distributed switch for management, vMotion, IP based storage and NSX network services. |
|  | vSAN Single-Rack HCI ESA Storage Model | Single-tier HCI architecture that utilizes high-performance flash storage devices. Alternative models conforming to the Blueprint Profile: - NFS Storage Model - Fibre Channel Storage Model |
|  | Simple NSX Manager Model | Addresses use cases requiring a minimal infrastructure footprint, lower scale, and lower availability requirements. |
|  | Host Fault Tolerant NSX Edge Cluster Model** | Minimalist footprint maintaining basic host-level availability |
| Physical Infrastructure | Single-Rack Network Fabric Model | Data center network fabric model for single-rack clusters. |

Management Domain Additional Cluster Design Selections [Optional]**

To use Self-Service Consumption via VCF Automation in this blueprint you need to deploy an additional vSphere cluster in your management domain. The following models for the addtional cluster align with the design profile for this blueprint.

| Layer | Selected Model | Additional Information |
| --- | --- | --- |
| Virtual Infrastructure | vSphere Single-Rack Cluster Model | vSphere cluster with all nodes in the same physical rack. Alternative models conforming to the Blueprint Profile: - Layer 2 Multi-Rack Cluster Model |
|  | Single Distributed Switch Model | Single vSphere distributed switch for management, vMotion, IP based storage and NSX network services. |
|  | vSAN Single-Rack HCI ESA Storage Model | Single-tier HCI architecture that utilizes high-performance flash storage devices. Alternative models conforming to the Blueprint Profile: - NFS Storage Model - Fibre Channel Storage Model |
|  | VPC with Full Services Workload Networking Model** | The model uses a primary (Tier-0) Gateway configured in Active/Standby mode. It provides a VPC-compatible workload domain with network connectivity established via a Centralized Transit Gateway. Alternative models conforming to the Blueprint profile: - NSX Segment Network Virtualization Scalable Workload Networking Model - VLAN backed Port Groups |
|  | Host Fault Tolerant NSX Edge Cluster Model | Minimalist footprint maintaining basic host-level availability |
| Physical Infrastructure | Single-Rack Network Fabric Model | Data center network fabric model for single-rack clusters. Selected models denoted with ** can also be omitted if VCF Automation is not being included in the deployment. This will result in using VLAN-backed networks for workloads. Alternatively the NSX Segment Network Virtualization Scalable Workload Networking Model can be used, which will also require the Host Fault-tolerant NSX Edge Cluster model. |

### Planning and Preparation for VCF Fleet in a Single Site with Minimal Footprint

Before you start the implementation of the VCF Fleet in a Single Site with Minimal Footprint Blueprint, you should follow the procedures in the table below to ensure a smooth deployment.

High Level Deployment Requirements

The following is a high level overview of the requirements to deploy this blueprint. To fully size and gather input requirements refer to the Planning and Preparation for the VCF Fleet section.

| Aspect | Requirements |
| --- | --- |
| Physical Hosts | - 4 for initial management domain cluster |
| Physical VLANs, Networks and IP allocations | 7x VLANS (with a physical network gateway interface attached) - 1x ESX Management Network (7 IPs) - 1 IP per host (x7 hosts) - 1x VM Management Network (16 IPs) - Fleet Management (1 IP) - VCF Operations (1 IP) - VCF Operations Collector (1 IP) - VCF Automation VIP (1 IP) ** - VCF Automation Node IPs (2 IPs) *, ** - NSX Manager (1 IP) - vCenter (1 IP) - SDDC Manager (1 IP) - Edge VMs (2 IPs) ** - Supervisor Control Plane (5 consecutive IPs) *, ** - 1x vSAN Network (or NFS if using NFS storage model) (7 IPs) - 1 IP per host (x7 hosts) - 1x vMotion Network (7 IPs) - 1 IP per host (x7 hosts) - 1x Host TEP Network (14 IPs) - 2 IPs per host (x7 hosts) - 1x Edge TEP Network (4 IPs) - 2 IPs per Edge VM (x2 Edge VMs) ** - 1x Edge Uplink Network (2 IPs) - 1 IP per Edge VM (x2 Edge VMs) ** Important All IP addresses should have forward and reverse DNS entries configured in your managed DNS server apart from those marked with a * |
| Virtual Networking ** | - Supervisor Service CIDR - e.g. 10.96.0.0/23 - VPC Private IP Block (Control Plane) - Distributed Transist Gateway Private IP Block (Control Plane) - External IP Block (Control Plane & Workload/Organizations) - Private VPC Block for each Organization’s VPCs - Private TGW IP Block for each Organization |

Planning and Preparation for the VCF Fleet

| Procedure Grouping | Procedure | Information |
| --- | --- | --- |
| Planning and Preparation | Download the Planning and Preparation Workbook. | The Planning and Preparation Workbook can be located on the Broadcom Technical Documentation site. |
| Prerequisites | Review and ensure compliance with the items listed in the Prerequisite Checklist tab of the Planning and Preparation Workbook. | This will ensure that the all hardware, network and infrastructural prerequisites are met before commencing deployment. |
| Sizing | In a distinct copy of the Planning and Preparation Workbook, populate the Management Domain Sizing tab. | This will ensure that the management domain is adequately sized to host your VCF Fleet deployment. |
| Management Domain | In the same Planning and Preparation Workbook, select the following options on the VCF & VVF Planning tab: - VMware Cloud FoundationDeployment Version: 9.0.0.0 - Deployment Specification: VMware Cloud FoundationInstance to perform operation on: First Instance - Operation to be performed: Deploy a new VCF fleet Populate the Management Domain Creation tab. | This will gather all the required physical network, IP Addressing and infrastructural details required to perform the deployment of the management domain. |

### VCF Fleet Design

This section describes the VCF Fleet Design.

#### Fleet Level Components on Shared Management Network Model

The fleet level management components can be deployed to the same vSphere Distributed Switch port group used for the VCF Instance management components, this can be achieved using VCF Installer during initial deployment of the VCF fleet.

Note	VCF Operations collector and VCF Operations for networks collectors are considered instance level components. It is recommended to always deploy these components to the management domain VM management port group.

Figure 3-2. Fleet Level Components on Shared Management Network Model in VMware Cloud Foundation

Fleet Level Components on Shared Management Network Model Attributes

A Fleet Level Components on Shared Management Network Model has the following attributes.

| Attribute | Detail |
| --- | --- |
| Connectivity Type | vSphere VLAN Port Group |
| Network Isolation | No, sharing the same VLAN as the VCF management domain components |
| NSX Edge Cluster | Only if NSX load balancer is required |
| Stretching Networks between availability zones | Physical network fabric must stretch the VLAN between availability zones |
| Stretching Networks between regions | Not suited to disaster recovery use case as network is shared with management domain components, failover testing not supported. |

Fleet Level Components on Shared Management Network Model Design Requirements

Table 3-1. Fleet Level Components on Shared Management Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-FLT-SM-REQD-001 | Use the management domain vCenter VM management port group for the Fleet level components | No additional network required | No logical network isolation |
| VCF-FLT-SM-REQD-002 | Use the same VLAN for Fleet level components and the VCF Instance managment VMs | No additional VLAN required | Separate physical firewall rules may not be possible with single VLAN |

Fleet Level Components on Shared Management Network Model Design Recommendations

Table 3-2. Fleet Level Components on Shared Management Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-FLT-SM-RCMD--001 | Deploy the fleet level components during initial deployment using VCF Installer. | Simplified deployment using UI. | None. |

#### VCF Fleet-Wide Single Sign-On Model

With this topology each VCF fleet contains a single VCF Identity Broker deployment. A user can log in once and gain VCF Single Sign-On capabilities for all components across all VCF instances in the same VCF fleet.

For each of these VCF Single Sign-On Models, both VCF Identity Broker Models are supported (embedded and appliance), however it is recommended to use the Appliance VCF Identity Broker Model for multi-VCF Instance Single Sign-On deployments due to availability and scale.

Figure 3-3. VCF Fleet-Wide Single Sign-On Model

Table 3-3. VCF Fleet-Wide Single Sign-On Model Attributes

| Attribute | Detail |
| --- | --- |
| Scale | Low Scale - A single VCF Identity Broker services all VCF Instances in your VCF fleet. |
| Resources requirements | Low Resource Footprint - A single VCF Identity Broker deployment is required for one VCF fleet. |
| Operational Overhead | Low Operational Overhead - A VCF Identity Broker per VCF fleet will need to be managed separately for operational tasks such as backup / restore. |
| Availability | Low Availability per Private Cloud - A single VCF Identity Broker servicing all VCF Instances in your VCF fleet. |
| Blast Radius | Large Blast Radius - An outage of one VCF Identity Broker appliance will impact logins for a single VCF fleet. |
| Single Sign-On scope | Large SSO Scope - Users will not need to re-authenticate when moving between different VCF instances within the same VCF fleet.. |

Table 3-4. VCF Fleet-Wide Single Sign-On Design Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implications |
| --- | --- | --- | --- |
| VCF-SSO-REQD-VFL-001 | Deploy VCF Identity Broker in the management domain of the first VCF Instance. | - Provides an authentication service for the first VCF Instance. - Provides VCF Single Sign-On capabilities to the first VCF Instance. | Workload domains will use the same authentication source as management domain |
| VCF-SSO-REQD-VFL-002 | Configure additional VCF Instances in the VCF fleet to consume the same VCF Identity Broker configured in the first VCF Instance. | - Provides an authentication source for all components in the VCF fleet. - Provides VCF Single Sign-On capabilities to all components in the VCF fleet. | An impact to the VCF Identity Broker service will impact the VCF fleet. |

### Physical Infrastructure Design

This section describes the Physical Infrastructure Design

#### Single-Rack Network Fabric Model

This section describes the VLAN and gateway requirements to support the vSphere Single-Rack Cluster Model.

The data center network fabric provides connectivity to the vSphere clusters. The vSphere Cluster Models and Storage Models chosen will determine which data center availability model is required to support the connectivity and availability within the cluster. This section describes the requirements to support a vSphere Single-Rack Cluster Model. Depending upon the Storage type chosen, additional VLAN requirements may apply.

Note	If this rack will contain NSX Edge Nodes, additional VLAN Requirements will apply. NSX Edge Cluster Options for Single Availability Zone

Table 3-5. Infrastructure VLAN Scope and First Hop Gateway Availability for Single Rack

| Traffic Type |  | Required VLAN Scope | First Hop Gateway Availability | MTU |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | minimum | preferred |
| Management VM Appliances* |  | Rack Local | Highly Available | 1500 | 1500 |
| ESX host | ESX management VMK * |  |  |  | 1500 |
|  | IP Storage VMKs (VSAN, NFS, iSCSI) |  |  |  | 9000 |
|  | vMotion VMK |  |  |  |  |
|  | Host TEP |  |  | 1700 |  |
| NSX Edge node | Depends on NSX Edge Cluster Models design chosen |  |  |  |  |
|  | Edge Node management | Rack Local | Highly Available | 1500 | 9000 |
|  | Edge TEP |  |  | 1700 |  |
|  | Edge RTEP |  |  | 1500 |  |
|  | Edge Uplink |  | Static Routing - Highly Available BGP - None | 1700* When using separate VLAN segments for management component VMs and ESX VMKs. |  |

Table 3-6. Workload VLAN Scope and First Hop Gateway Availability for Single Rack

| Traffic Type | Required VLAN Scope | First Hop Gateway Availability | MTU |  |
| --- | --- | --- | --- | --- |
|  |  |  | minimum | preferred |
| VLAN segment for workload VMs | Rack Local | Highly Available | 1500 | 9000 |
| VLAN segment for DTGW | Rack Local | Highly Available | 1500 | 9000 |

### Virtual Infrastructure Design

This section describes the Virtual Infrastructure Design.

#### Management Domain Model

The management domain in a VCF fleet hosts the management components of the VCF Instance where it is deployed and the management components of the fleet.

Figure 3-4. Management Domain Model in VMware Cloud Foundation

Management Domain Model Attributes

The Management Domain Model has the following attributes.

| Attribute | Details |
| --- | --- |
| Resource allocation | - VCF fleet management components. - VCF Instance components. - NSX Edge nodes. - Can also run business workloads. |
| VCF fleet management components | - The management domain in the first VCF Instance contains the following fleet-level management components: - VCF Operations - VCF Automation - Management domains in additional VCF Instances contain the following instance-level management components: - VCF Operations collector(s) |
| VCF Instance management components | - Contains the following management appliances for all workload domains: - ESX hosts - vCenter - NSX Manager (dedicated) - SDDC Manager |

Management Domain Model Options

According to your objectives, the Management Domain Model can have options from the following design areas.

| Design Area | Options |
| --- | --- |
| Cluster models | - First cluster: - vSphere Single-Rack Cluster Model - Layer 2 Multi-Rack Cluster Model - vSphere Stretched Cluster Model (Day 2) - Additional clusters: - All cluster models. |
| VCF Operations models | - Simple VCF Operations Model - High Availability VCF Operations Model |
| VCF Automation models | - Simple VCF Automation Model - High Availability VCF Automation Model |
| NSX Manager models | - Simple NSX Manager Model - High Availability NSX Manager Model |

Management Domain Model Requirements

Table 3-7. Management Domain Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-WLD-REQD-CFG-001 | Deploy a management domain to host all management components for the VCF instance. | - Guarantees sufficient resources for management components. - Enables the use of specific hardware to meet only the needs of the management components. - Enables the use of dedicated physical compute, network and storage separately from those used for additional workloads. - Enables separate life cycle management of management and workload components. | - You must carefully size the management domain to accommodate planned deployments of additional workload domains and management components. - Hardware might not be fully utilized until full-scale deployment has been reached. |

Management Domain Model Recommendations

Table 3-8. Management Domain Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-WLD-RCMD-CFG-001 | If running business workloads in the management domain, configure the following vSphere resource pools to control resource usage by management and business workloads. - cluster-name-rp-sddc-mgmt - cluster-name-rp-sddc-edge - cluster-name-rp-user-edge - cluster-name-rp-user-vm | Ensures sufficient resources for the management components. | You must manually create the vSphere resource pools and manage their settings over time. |

#### ESX Detailed Design

In the design of the ESX host configuration for your VMware Cloud Foundation platform, consider the resources, networking, and security policies that are required to support the virtual machines in each workload domain cluster.

ESX Hardware and Sizing Considerations

For more information on host sizing, refer to the Planning and Preparation workbook, and the VMware Cloud Foundation Sizer.

Table 3-9. ESX Hardware and Sizing Considerations

| Hardware Element | Considerations |
| --- | --- |
| BIOS/Firmware | Ensure the ESX host hardware is on the HCL and that you are running the latest supported BIOS & firmware. |
| CPU | - Total CPU requirements for the workloads that are running in the cluster. - Host failure and maintenance scenarios. Keep the over-commitment ratio vCPU-to-pCPU less than or equal to 2:1 for the management domain and less than or equal to 8:1 for workload domains. - Number of physical cores, not logical cores. - Simultaneous multi-threading (SMT) in CPUs improve CPU performance however the performance enhancement will not be equivalent to 100% more CPU power. |
| Memory | - Total memory requirements for the workloads that are running in the cluster. - When sizing memory for the ESX hosts in a cluster, to reserve the resources of one host for failover or maintenance, set the admission control setting to N+1, which reserves the resources of one host for failover or maintenance. - vSAN OSA. - To support the maximum number of disk groups, you must provide 32 GB of RAM. - vSAN ESA. - You must provide 512 GB of RAM. |
| Storage | - Use high-endurance devices such as a hard drive or SSD for boot device. - Use a 128GB or greater boot device to maximize the space available for ESX-OS Data. - Use hosts with homogeneous configuration. |
| Network | - A single NIC connected to a single switch is supported. - Use two or more pairs of NICs connected to multiple ToR switches is recommended to ensure redundancy. - 25GbE is recommended |
| Power | - Use redundant power supplies, preferably connected to separate circuits to ensure availability. - Consider the impact of enabling Power Management on your ESX hosts. In order to allow ESX to control CPU power-saving features you need to set power management in the BIOS to OS Controlled Mode. |

ESX Design Requirements and Recommendations

In your ESX design, you apply certain best practices.

Table 3-10. ESX Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-ESX-REQD-CFG-001 | Install no less than the minimum number of ESX hosts required for the vSphere cluster model being deployed. | - Ensures that availability requirements are met. - If one of the ESX hosts is not available because of a failure or maintenance event, the CPU over-commitment ratio becomes 2:1. | None. |
| VCF-ESX-REQD-CFG-002 | Ensure that each ESX host matches the required CPU, memory, and storage specification. | Ensures that workloads will run without contention even during failure and maintenance conditions. | Assemble the server specification and number according to the sizing in the VMware Cloud Foundation Planning and Preparation Workbook which is based on projected deployment size. |
| VCF-ESX-REQD-SEC-001 | Regenerate the certificate of each ESX host after assigning the ESX host an FQDN. | Establishes a secure connection with the VCF Installer during the initial deployment and prevents man-in-the-middle (MiTM) attacks. | You must manually regenerate the certificates of the ESX hosts before the deployment. |

Table 3-11. ESX Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-ESX-RCMD-CFG-001 | When using vSAN as primary storage in the management domain, use vSAN ReadyNodes for each ESX host. | Your management domain is fully compatible with vSAN at deployment. Note For information about the models of physical servers that are vSAN-ready, see vSAN Compatibility Guide for vSAN ReadyNodes. | - Hardware choices might be limited. - If you plan to use a server configuration that is not a vSAN ReadyNode, your CPU, disks, and I/O modules must be listed on the Broadcom Compatibility Guide under CPU Series and vSAN Compatibility List aligned to the ESX version. |
| VCF-ESX-RCMD-CFG-002 | Allocate ESX hosts with uniform configuration across the default management vSphere cluster. | A balanced vSphere cluster has these advantages: - Predictable performance even during hardware failures. - Minimal impact of resynchronization or rebuild operations on performance. | You must apply vendor sourcing, budgeting, and procurement considerations for uniform server nodes on a per cluster basis. |
| VCF-ESX-RCMD-CFG-003 | When sizing CPU, do not consider multi-threading technology and associated performance gains. | Although multi-threading technologies increase CPU performance, the performance gain depends on running workloads and differs from one case to another. | Because you must provide more physical CPU cores, costs increase and hardware choices become limited. |
| VCF-ESX-RCMD-CFG-004 | Install and configure all ESX hosts in the default management vSphere cluster to boot using a 128-GB device or larger. | Provides ESX hosts that have large memory, that is, greater than 512 GB, with enough space for the scratch partition when using vSAN. | None. |
| VCF-ESX-RCMD-CFG-005 | Use the default configuration for the scratch partition on all ESX hosts in the default management vSphere cluster. | - If a failure in the vSAN cluster occurs, the ESX hosts remain responsive and log information is still accessible. - It is not possible to use a vSAN datastore for the scratch partition. | None. |
| VCF-ESX-RCMD-CFG-006 | For workloads running in the default management vSphere cluster, save the virtual machine swap file at the default location. | Simplifies the configuration process. | Increases the amount of replication traffic for management workloads that are recovered as part of the disaster recovery process. |
| VCF-ESX-RCMD-NET-001 | Place the ESX hosts in each management domain vSphere cluster on the ESX management network that is separate from the VM management network. | - Enables the separation of the physical VLAN between ESX hosts and the other management components for security reasons. - The VM management network is not required for a multi-rack compute-only cluster in a workload domain. | Increases the number of VLANs required. |
| VCF-ESX-RCMD-NET-002 | Place the ESX hosts in each workload domain on a separate ESX host management VLAN-backed network. | Enables the separation of the physical VLAN between the ESX hosts in different workload domains for security reasons. | Increases the number of VLANs required. For each workload domain, you must allocate a separate management subnet. |
| VCF-ESX-RCMD-NET-003 | Use dedicated VLANs for each traffic type. | Enables the separation of the physical VLAN for vSAN, vMotion, NFS & backup traffic. | Increases the number of VLANs required. For each traffic type you must allocate a separate VLAN and subnet. |
| VCF-ESX-RCMD-SEC-001 | Set the advanced setting UserVars.SuppressShellWarning to 0 across all ESX hosts in the management domain. | - Ensures compliance with the vSphere Security Configuration Guide and with security best practices - Enables the warning message that appears in the vSphere client every time SSH access is activated on an ESX host. | You must turn off SSH enablement warning messages manually when performing troubleshooting or support activities. |

#### vCenter Detailed Design

The vCenter design considers the location, size, high availability, and identity domain isolation of the vCenter instances for the workload domains in a VCF fleet.

vCenter Attributes

A vCenter has the following attributes.

Table 3-12. vCenter Attributes

| Design Area | Attributes |
| --- | --- |
| vCenter Deployment Type | - One vCenter instance for the management domain that manages the management components of the VCF Instance - Optionally, additional vCenter instances for workload domains to support workloads. - vSphere HA protecting all vCenter appliances. |
| vCenter Single Sign-On | - Each vCenter has a distinct vCenter Single Sign-On domain. - Can be linked for sharing of tags using vCenter Linking Groups in VCF Operations. |

Methods for Protecting vCenter

VMware Cloud Foundation supports multi-rack management vSphere clusters, vSAN stretched clusters, or 3rd party synchronous replication solutions such as metro clusters that are designed to provide disaster recovery and/or high availability for the entire management stack (vCenter, NSX Manager, etc.) as opposed to a single vCenter.

Table 3-13. Methods for Protecting vCenter

| High Availability Method | Supported in VMware Cloud Foundation | Considerations |
| --- | --- | --- |
| vSphere High Availability | Yes | - You must ensure sufficient resources in the cluster to allow VMs to restart on another host in the event of a host failure |
| vCenter High Availability (vCenter HA) | No | - vCenter services must fail over to the passive node so there is no continuous availability. - Recovery time can be up to 15 mins. - You must meet additional networking requirements for the private network. - vCenter HA requires additional resources for the passive and witness nodes. - Lifecycle management is complicated because you must manually delete and recreate the standby virtual machines during a lifecycle management operation. |
| vSphere Fault Tolerance (vSphere FT) | No | - The vCPU limit of vSphere FT vCPU would limit vCenter appliance size to medium. - You must provide a dedicated network. |

vCenter Design Requirements

Table 3-14. vCenter Design Requirements for VMware Cloud Foundation

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VCS-REQD-CFG-001 | Deploy a dedicated vCenter appliance for the management domain of the VCF Instance. | - Isolates vCenter failures to management or customer workloads. - Isolates vCenter operations between management and business workloads. - Supports a scalable vSphere cluster design where you can reuse the management components as more customer workloads are added to the VCF Instance . - Simplifies capacity planning for customer workloads because you do not consider management workloads for the workload domain vCenter. - Improves the ability to upgrade the vSphere environment and related components by enabling for explicit separation of maintenance windows: - Management workloads remain available while you are upgrading the tenant workloads. - Workloads remain available while you are upgrading the management nodes. - Supports clear separation of roles and responsibilities to ensure that only administrators with granted authorization can control the management workloads. - Facilitates quicker troubleshooting and problem resolution. - Simplifies disaster recovery operations by supporting a clear separation between recovery of the management components and tenant workloads. - Provides isolation of potential network issues by introducing network separation of the vSphere clusters in the VCF Instance. | Requires a separate license for the vCenter instance in the management domain |
| VCF-VCS-REQD-NET-001 | Place all workload domain vCenter appliances on the VM management network in the management domain. | - Simplifies IP addressing for management VMs by using the same VLAN and subnet. - Provides simplified secure access to management VMs in the same VLAN network. | None. |
| VCF-VCS-REQD-SSO-001 | Create all vCenter instances within a VCF Instance in their own unique vCenter Single Sign-On domain. | - Sharing vCenter Single Sign-On domains is not supported. - Enables isolation at the vCenter Single Sign-On domain layer for increased security separation. - Supports up to 25 workload domains. | - Each vCenter instance is managed through its own pane of glass using a different set of administrative credentials. - You must manage password rotation for each vCenter Single Sign-On domain separately. |

Table 3-15. vCenter Design Recommendations for VMware Cloud Foundation

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VCS-RCMD-CFG-001 | Deploy an appropriately sized vCenter appliance for each workload domain. | Ensures resource availability and usage efficiency per workload domain. | You must ensure adequate compute resources are available to run all vCenter appliances. |
| VCF-VCS-RCMD-CFG-002 | Deploy a vCenter appliance with the appropriate storage size. | Ensures resource availability and usage efficiency per workload domain. | You must ensure adequate storage resources are available to run all vCenter appliances. |
| VCF-VCS-RCMD-CFG-003 | Protect workload domain vCenter appliances by using vSphere HA. | vSphere HA is the only supported method to protect vCenter availability in VCF. | vCenter becomes unavailable during a vSphere HA failover. |
| VCF-VCS-RCMD-CFG-004 | In vSphere HA, set the restart priority policy for the vCenter appliance to high. | vCenter is the management and control plane for physical and virtual infrastructure. In a vSphere HA event, to ensure the rest of the SDDC management stack comes up faultlessly, the workload domain vCenter must be available first, before the other management components come online. | If the restart priority for another virtual machine is set to highest, the connectivity delay for the management components will be longer. |

#### vSphere Single-Rack Cluster Model

The Single-Rack Cluster Model is the simplest form of protection where workloads are deployed to a vSphere cluster in the same physical rack, in a single availability zone.

The Single-Rack Cluster Model relies on vSphere HA to protect workloads against host failures.

Figure 3-5. Single-Rack Cluster Model

Single-Rack Cluster Model Attributes

A Single-Rack Cluster Model has the following attributes.

| Attribute | Detail |
| --- | --- |
| Datacenters | Single datacenter. |
| Physical rack mapping | All cluster hosts exist in one physical rack. |
| Resilience | vSphere HA protects against host failures. |

Single-Rack Cluster Model Sizing Considerations

You choose the number of ESX hosts per vSphere cluster based on storage type and workload domain considerations.

| Attribute | Storage Type | VCF Installer Deployment Model | Management Domain (Default Cluster) | Reserved capacity for handling ESX host failures per cluster | Management Domain (Additional Clusters) or Workload Domain (All Clusters) | Reserved capacity for handling ESX host failures per cluster |
| --- | --- | --- | --- | --- | --- | --- |
| Minimum number of ESX hosts | vSAN | Simple | Three (3) | - 33% CPU and memory - Tolerates one host failure | Three (3) | - 33% CPU and memory - Tolerates one host failure |
|  |  | High Availability | Four (4) | - 25% CPU and memory - Tolerates one availability zone failure |  |  |
|  | NFS or FC | Simple | Two (2) | - 50% CPU and memory - Tolerates one availability zone failure | Two (2) | - 50% CPU and memory - Tolerates one availability zone failure |
|  |  | High Availability | Four (4) | - 25% CPU and memory - Tolerates one availability zone failure |  |  |

Important	vSphere clusters are configured allowing for the failure of a single ESX host while still having enough resources to run all vSphere cluster workloads.

Attention	* Designs should be completed in consultation with your storage vendor.

Single-Rack Cluster Model Options

A Single-Rack Cluster Model can leverage options from the design areas listed below, depending on objectives.

| Design Area | Options |
| --- | --- |
| Distributed switch models | - Single Distributed Switch Model - Storage Separation Distributed Switch Model - Workload Separation Distributed Switch Model - Storage and Workload Separation Distributed Switch Model |
| Storage models (principal) | - vSAN - vSAN Single-Rack HCI ESA Storage Model - vSAN Single-Rack HCI OSA Storage Model - vSAN Single-Rack Storage Cluster Storage Model - vSAN Single-Rack Compute Cluster Storage Model - Fibre Channel Storage Model - NFS Storage Model |

Single-Rack Cluster Model Design Requirements

Table 3-16. Common vSphere Cluster Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-CLS-REQD-CFG-001 | Create a vSphere cluster in each workload domain for the initial set of ESX hosts. | - Simplifies configuration by isolating management from customer workloads. - Ensures that customer workloads have no impact on the management stack. | Management of multiple vSphere clusters and vCenter instances increases operational overhead. |
| VCF-CLS-REQD-CFG-002 | Allocate a minimum number of ESX hosts according to the vSphere cluster type being deployed. | Ensures correct level of redundancy to protect against host failure in the vSphere cluster. | To support redundancy, you must allocate additional ESX host resources. |
| VCF-CLS-REQD-CFG-003 | Use vSphere Lifecycle Manager images as the life cycle management method for all vSphere clusters. Note Imported workload domains may be using vSphere Lifecycle Manager baselines. It is recommended to transition them to use vSphere Lifecycle Manager images. | vSphere Lifecycle Manager images simplify the management of firmware and vendor add-ons manually. | - A vSphere Lifecycle Manager cluster image is required during workload domain or vSphere cluster deployment. |
| VCF-CLS-REQD-CFG-004 | Use vSphere HA to protect all virtual machines against failures. | vSphere HA supports a robust level of protection for both ESX host and virtual machine availability. | You must provide sufficient resources on the remaining ESX hosts so that virtual machines can be restarted on those hosts in the event of an ESX host outage. |

Table 3-17. Common vSphere Cluster Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-CLS-RCMD-CFG-001 | Configure admission control for one (1) ESX host failure and percentage-based failover capacity. | - Using the percentage-based reservation works well in situations where virtual machines have varying and sometimes significant CPU or memory reservations. - vSphere automatically calculates the reserved percentage according to the number of ESX host failures to tolerate and the number of ESX hosts in the vSphere cluster. | In a cluster of four (4) ESX hosts, the resources of only three (3) ESX hosts are available for use. |
| VCF-CLS-RCMD-CFG-002 | Enable VM Monitoring for each vSphere cluster. | VM Monitoring provides in-guest protection for most VM workloads. The application or service running on the virtual machine must be capable of restarting successfully after a reboot or the virtual machine restart is not sufficient. | None. |
| VCF-CLS-RCMD-CFG-003 | Enable vSphere DRS on all vSphere clusters, using the default fully automated mode with medium threshold. | Provides the best trade-off between load balancing and unnecessary migrations with vMotion. | If a vCenter outage occurs, the mapping from virtual machines to ESX hosts might be difficult to determine. |
| VCF-CLS-RCMD-CFG-004 | Enable Enhanced vMotion Compatibility (EVC) on all vSphere clusters in the management domain. | Supports vSphere cluster upgrades without virtual machine downtime. | - You must enable EVC only if the vSphere clusters contain ESX hosts with CPUs from the same vendor. - You must enable EVC on the default management domain vSphere cluster during bringup using the API and a JSON spec. |
| VCF-CLS-RCMD-CFG-005 | Set the vSphere cluster EVC mode to the highest available baseline that is supported for the lowest CPU architecture on the ESX hosts in the vSphere cluster. | Supports vSphere cluster upgrades without virtual machine downtime. | None. |
| VCF-CLS-RCMD-CFG-006 | If running business workloads in the management domain, configure the following vSphere resource pools to control resource usage by management and business workloads. - cluster-name-rp-sddc-mgmt - cluster-name-rp-sddc-edge - cluster-name-rp-user-edge - cluster-name-rp-user-vm | Ensures sufficient resources for the management components. | You must manually create the vSphere resource pools and manage their settings over time. |
| VCF-CLS-RCMD-CFG-007 | Use vSphere Cluster Services (vCLS) Retreat Mode. | System managed vCLS mode is deprecated. | You must manually change the vCLS mode. |

Single-Rack Cluster Model Design Recommendations

In your vSphere cluster design, you can apply certain best practices for standard clusters.

Table 3-18. Common vSphere Cluster Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-CLS-RCMD-CFG-001 | Configure admission control for one (1) ESX host failure and percentage-based failover capacity. | - Using the percentage-based reservation works well in situations where virtual machines have varying and sometimes significant CPU or memory reservations. - vSphere automatically calculates the reserved percentage according to the number of ESX host failures to tolerate and the number of ESX hosts in the vSphere cluster. | In a cluster of four (4) ESX hosts, the resources of only three (3) ESX hosts are available for use. |
| VCF-CLS-RCMD-CFG-002 | Enable VM Monitoring for each vSphere cluster. | VM Monitoring provides in-guest protection for most VM workloads. The application or service running on the virtual machine must be capable of restarting successfully after a reboot or the virtual machine restart is not sufficient. | None. |
| VCF-CLS-RCMD-CFG-003 | Enable vSphere DRS on all vSphere clusters, using the default fully automated mode with medium threshold. | Provides the best trade-off between load balancing and unnecessary migrations with vMotion. | If a vCenter outage occurs, the mapping from virtual machines to ESX hosts might be difficult to determine. |
| VCF-CLS-RCMD-CFG-004 | Enable Enhanced vMotion Compatibility (EVC) on all vSphere clusters in the management domain. | Supports vSphere cluster upgrades without virtual machine downtime. | - You must enable EVC only if the vSphere clusters contain ESX hosts with CPUs from the same vendor. - You must enable EVC on the default management domain vSphere cluster during bringup using the API and a JSON spec. |
| VCF-CLS-RCMD-CFG-005 | Set the vSphere cluster EVC mode to the highest available baseline that is supported for the lowest CPU architecture on the ESX hosts in the vSphere cluster. | Supports vSphere cluster upgrades without virtual machine downtime. | None. |
| VCF-CLS-RCMD-CFG-006 | If running business workloads in the management domain, configure the following vSphere resource pools to control resource usage by management and business workloads. - cluster-name-rp-sddc-mgmt - cluster-name-rp-sddc-edge - cluster-name-rp-user-edge - cluster-name-rp-user-vm | Ensures sufficient resources for the management components. | You must manually create the vSphere resource pools and manage their settings over time. |
| VCF-CLS-RCMD-CFG-007 | Use vSphere Cluster Services (vCLS) Retreat Mode. | System managed vCLS mode is deprecated. | You must manually change the vCLS mode. |

Table 3-19. Common vSphere Cluster Virtual Networking Infrastructure Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VNI-RCMD-CFG-001 | Use static IP pools to assign IP addresses to the host TEP interfaces. | - Removes the need for an external DHCP server for the host overlay VLANs. - You can use NSX Manager to verify static IP pool configurations. | None. |
| VCF-VNI-RCMD-CFG-002 | Create an uplink profile with a load balance source teaming policy with all active uplinks for ESX hosts. | For increased resiliency and performance, supports the concurrent use of more than one physical NICs on the ESX hosts that are configured as transport nodes. | None. |

#### Single Distributed Switch Model

A Single Distributed Switch Model in VMware Cloud Foundation provides a minimal deployment of networking using a single vSphere distributed switch for all network services such as management, vMotion, IP based storage and, NSX.

A Single Distributed Switch Model provides connectivity using the lowest number of physical NICs for each ESX host in the vSphere cluster while still providing availability from a single physical NIC failure, the network services running on the vSphere cluster will share the bandwidth provided by two physical NICs.

The Single Distributed Switch Model can be suited to vSphere clusters where the number of physical NICs must be reduced and the overall bandwidth requirements does not exceed that provided by two physical NICs. The bandwidth of the ESX host physical NICs will determine the network performance that can be achieved for the vSphere cluster.

Figure 3-6. Single Distributed Switch Model with 2 Physical NICs per ESX Host

Single Distributed Switch Model Attributes

A Single Distributed Switch Model has the following attributes.

| Attribute | Detail |
| --- | --- |
| Number of vSphere Distributed Switches | Single |
| Number of Physical NICs per ESX Host | Two (2) |
| Bandwidth | Shared for all network traffic |
| Network I/O Control | Enabled by default |

Single Distributed Switch Model Options

A Single Distributed Switch Model can leverage options from the design areas listed below, depending on objectives.

| Design Area | Options |
| --- | --- |
| Physical NIC Level Redundancy (NLR) | - Use a single 2-port NIC (no NLR). - Use two separate 2-port NICs. - Each traffic type is spread across two separate NIC to avoid single point of failure. - Results in unused ports on NIC. |
| Load Balancing Option | - Route based on physical NIC load. - Route based on the originating virtual port. - Use explicit failover order. - Route based on IP hash. |
| Data Path Mode | - Standard. - Enhanced Datapath Standard (Default). - Enhanced Datapath Dedicated. |

Common Distributed Switch Design Requirements

Table 3-20. Common Distributed Switch Design Requirements

| Design Requirements ID | Design Requirements | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VDS-COM-REQD-CFG-001 | Do not share a vSphere Distributed Switch across different vSphere clusters. (This applies to a greenfield deployment of a cluster using VCF automated workflows) | - Enables independent lifecycle management of vSphere Distributed Switch per vSphere cluster. - Reduces the size of the fault domain. | For multiple vSphere clusters, you manage more vSphere Distributed Switches. |

Common Distributed Switch Design Recommendations

Table 3-21. Common Distributed Switch Design Recommendations

| Design Recommendations ID | Design Recommendations | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VDS-COM-RCMD-CFG-001 | Configure the MTU size of the vSphere Distributed Switch to 9000 bytes for jumbo frames. | - Supports the MTU size required by system traffic types. - Improves traffic throughput. | When adjusting the MTU packet size, you must also configure the entire network path (VMkernel ports, virtual switches, physical switches, and routers) to support the same MTU packet size. |
| VCF-VDS-COM-RCMD-CFG-002 | For NSX enabled vSphere Distributed Switch with Overlay or VLAN transport zone configure Enhanced Datapath Standard. | Provides the best performance for bandwidth and packets per second for the edge nodes running on the cluster. | The physical NIC must support the Enhanced Datapath - Interrupt Mode feature. |
| VCF-VDS-COM-RCMD-DPG-001 | Use ephemeral port binding for the VM management distributed port group (default setting). | - Using ephemeral port binding provides the option to recover the vCenter instance that is managing the vSphere Distributed Switch. - The VM management network is not required for a multi-rack compute-only cluster in a VI workload domain | Port-level permissions and controls are lost across power cycles, and no historical context is saved. |
| VCF-VDS-COM-RCMD-DPG-002 | Use static port binding for all non-management distributed port groups (default setting). | Static binding ensures a virtual machine connects to the same port on the vSphere Distributed Switch. This allows for historical data and port-level monitoring. | Traffic not always deterministic, can be configured to another teaming policy during or after deployment if required. |
| VCF-VDS-COM-RCMD-DPG-003 | Use the Route based on physical NIC load teaming algorithm for the VM management distributed port group (default setting). | Reduces the complexity of the network design, increases resiliency, and can adjust to fluctuating workloads. | Traffic not always deterministic, can be configured to another teaming policy during or after deployment if required. |
| VCF-VDS-COM-RCMD-DPG-004 | Use the Route based on physical NIC load teaming algorithm for the ESX management distributed port group (default setting). | Reduces the complexity of the network design, increases resiliency, and can adjust to fluctuating workloads. | Traffic not always deterministic, can be configured to another teaming policy during or after deployment if required. |
| VCF-VDS-COM-RCMD-DPG-005 | Use the Route based on physical NIC load teaming algorithm for the vMotion distributed port group (default setting). | Reduces the complexity of the network design, increases resiliency, and can adjust to fluctuating workloads. | Traffic not always deterministic, can be configured to another teaming policy during or after deployment if required. |

Single Distributed Switch Model Design Requirements

Table 3-22. Single Distributed Switch Model Design Requirements

| Design Requirements ID | Design Requirements | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VDS-SDS-REQD-CFG-001 | Use a single vSphere Distributed Switch per vSphere cluster. | Reduces the complexity of the network design. | Reduces the number of vSphere Distributed Switches that must be managed per vSphere cluster. |
| VCF-VDS-SDS-REQD-CFG-002 | Create the following distributed port groups on the vSphere Distributed Switch: - VM management (mgmt domain only) - ESX management - vMotion - Storage | Provides the required network services for the vSphere cluster. | VM management distributed port group needs to be manually created on additional vSphere clusters in the management domain and any workload domain vSphere clusters. |
| VCF-VDS-SDS-REQD-CFG-003 | For Layer 3 multi-rack cluster deployments create the following distributed port groups on the vSphere Distributed Switch for each additional rack: - ESX management - vMotion - Storage (vSAN/NFS) | - Provides separate networks per rack in a multi-rack deployment. - Storage traffic has a separate isolated layer 2 broadcast domain per rack | Additional port groups will be created for each additional rack |
| VCF-VDS-SDS-REQD-CFG-004 | For stretched cluster deployments create the following distributed port groups on the vSphere Distributed Switch for availability zone 2 - ESX management - vMotion - vSAN | - Provides separate networks per rack in a multi-rack deployment. - vSAN traffic has a separate isolated layer 2 broadcast domain per availability zone | Additional port groups will be created for availability zone 2 |

Single Distributed Switch Model Design Recommendations

Table 3-23. Single Distributed Switch Model Design Recommendations

| Design Recommendations ID | Design Recommendations | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VDS-SDS-RCMD-DPG-001 | Use the Route based on physical NIC load teaming algorithm for any storage distributed port group (default setting). | Reduces the complexity of the network design, increases resiliency, and can adjust to fluctuating workloads. | Traffic not always deterministic, can be configured to another teaming policy during or after deployment if required. |
| VCF-VDS-SDS-RCMD-NIO-001 | Enable Network I/O Control on vSphere Distributed Switch (default setting). | Increases resiliency and performance when network traffic shares uplinks. | Network I/O Control might impact network performance for critical traffic types if misconfigured. |
| VCF-VDS-SDS-RCMD-NIO-002 | Set the share value for management traffic to Normal (default setting). | By keeping the default setting of Normal, management traffic is prioritized higher than vMotion but lower than vSAN traffic. Management traffic is important because it ensures that the hosts can still be managed during times of network contention. | None. |
| VCF-VDS-SDS-RCMD-NIO-003 | Set the share value for vMotion traffic to Low (default setting). | During times of network contention, vMotion traffic is not as important as virtual machine or storage traffic. | During times of network contention, vMotion takes longer than usual to complete. |
| VCF-VDS-SDS-RCMD-NIO-004 | Set the share value for virtual machines to High (default setting). | Virtual machines are the most important asset in a VCF Instance. Leaving the default setting of High ensures that they always have access to the network resources they need. | None. |
| VCF-VDS-SDS-RCMD-NIO-005 | Set the share value for principal IP-based storage traffic to High (default setting). | During times of network contention, storage traffic needs guaranteed bandwidth to support virtual machine performance. | None. |
| VCF-VDS-SDS-RCMD-NIO-006 | All other traffic types are set using default settings. | Other traffic types are only added during day 2 operations. | Can be manually adjusted on day 2 if required. |

#### vSAN Single-Rack HCI ESA Storage Model

vSAN ESA uses a single-tier architecture that utilizes high-performance flash storage devices. vSAN ESA supports only NVMe high performance storage devices.

Figure 3-7. vSAN Single-Rack HCI ESA Storage Model

Table 3-24. vSAN Single-Rack HCI ESA Storage Model Attributes

| Attribute | Detail |
| --- | --- |
| Single Tier | All data is stored on the same type of storage device, regardless of access frequency or importance. |
| Storage Pool | All NVMe Flash Storage. |
| Data Services | - Compression using Storage Policy (default). - Encryption. - Integrated File Services. - vSAN iSCSI Target Service. - vSAN storage clusters - Stretched Cluster - Fault Domains - Kubernetes persistent storage services (block and file). - Native scaleable snapshots |

Table 3-25. vSAN Single-Rack HCI ESA Storage Model Options

| Design Area | Options |
| --- | --- |
| Distributed switch models | - Single Distributed Switch Model - Workload Separation Distributed Switch Model - Storage Separation Distributed Switch Model - Storage and Workload Separation Network Model |
| vSAN ESA Cluster models | - vSAN Single-Rack HCI ESA Storage Model (current model) - vSAN ESA Storage Cluster Storage Model - vSAN ESA Stretched Storage Cluster - vSAN Stretched HCI Cluster Storage Model - vSAN HCI Multi-Rack Model - vSAN HCI Datastore Sharing Model |

Table 3-26. Common vSAN Design Requirements for All Models

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-REQD-CFG-001 | Minimum vSAN cluster size. | Minimum valid vSAN HCI cluster is three (3) nodes all contributing to storage. | - Supports simple install of a management domain default vSphere cluster only. - A three (3) node vSAN cluster will not offer automatic rebuild of components in the event of a failure. - Using a smaller cluster limits the workload that can be placed on a vSAN cluster. Important vSAN Storage Clusters require minimum four (4) vSAN Nodes see vSAN Single-Rack Storage Cluster Storage Model for more details |
| VCF-VSAN-REQD-CFG-002 | Provide sufficient raw capacity to meet the initial needs of the workload domain vSphere cluster. | Ensures that sufficient resources are present to create the workload domain vSphere cluster. | Requires determining the workload capacity requirements prior to deployment. Note Refer to the VMware Cloud Foundation Planning and Preparation Workbook for accurate sizing guidance. |
| VCF-VSAN-REQD-CFG-003 | vSphere Lifecycle Management (vLCM) image based management is required for vSAN clusters. | - All new vSAN clusters, irrespective of OSA or ESA architectures, are required to be using vSphere Lifecycle Manager images for VMware Cloud Foundation. - As per VCF-CLS-REQD-CFG-003 vSphere Lifecycle Manager images simplify the management of firmware and vendor add-ons. | Imported vSAN OSA clusters may use vLCM baselines, but have to be converted prior to upgrade to VMware Cloud Foundation 9.0. |
| VCF-VSAN-REQD-CFG-004 | Verify the hardware components used for vSAN deployments are on the vSAN hardware compatibility list. | Helps prevent hardware related issues during workload deployment and operation. | - Requires validating vSAN hardware to ensure its on the compatibility list prior to deployment. - Restricts the number of devices that can be used to a certified list. Note Refer to the vSAN hardware compatibility guide for certified hardware https://compatibilityguide.broadcom.com/. |
| VCF-VSAN-REQD-CFG-005 | Do not use NVMe storage devices attached to a Tri-Mode controller for vSAN diskgroups or storage pools. | - Discrete RAID controllers that support SATA/SAS/NVMe are often known as "Tri-mode controllers". - VMware vSAN will only support SAS and SATA devices attached to a Tri-mode controller. - NVMe devices attached to Tri-mode controller is NOT a vSAN supported configuration. Note Refer to knowledge base article KB314305 | - NVMe devices are only supported directly connected to a PCIe slot on the bus. - May require to work with server vendor to verify or modify configurations. |
| VCF-VSAN-REQD-CFG-006 | For vSAN HCI clusters, excluding vSAN storage clusters, configure the vSAN network gateway IP address as the isolation address for the cluster. | vSphere HA can validate against an IP address on the vSAN network if a host is isolated. | Allocate an additional IP address. Top of rack switches may configure a port for switch virtual interface (SVI) which can be used as the isolation address. See VCF-VSAN-NET-REQD-CFG-003 |
| VCF-VSAN-REQD-CFG-007 | For vSAN HCI clusters, excluding vSAN storage clusters, set the advanced cluster setting das.usedefaultisolationaddress to false. | vSphere HA in vSAN uses the ESX hosts management network default gateway IP for isolation detection when configured with default settings. This setting avoids conflicts if vSphere HA and vSAN triggers different responses when failures occur. | May require to configure a SVI (Switched Virtual Interface) gateway address for the VLAN assigned to the vSAN network. |

Table 3-27. Common vSAN Design Recommendations for All Models

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-RCMD-CFG-001 | It is recommended to start with four (4) vSAN nodes. | - This ensures vSAN objects will automatically rebuild in the case of a failure or maintenance event. - This allows enabling Reserved Capacity on four (4) node clusters. - This allows space efficient storage polices to be automatically configured when used with vSAN ESA clusters. | This will potentially increase cost and hardware footprint of vSAN cluster. |
| VCF-VSAN-RCMD-CFG-002 | Include TPM (Trusted Platform Module) in the hardware configuration of the ESX hosts. | Ensures that the keys issued to the hosts in a vSAN cluster using Data-at-Rest Encryption are cryptographically stored on a TPM device. | Can limit the choice of hardware configurations. |
| VCF-VSAN-RCMD-CFG-003 | Enable Reserved Capacity on vSAN clusters. | - Reserved Capacity will implement a sliding scale storage capacity resource reservation as the vSAN node count increases per vSAN cluster. For example, the Host Rebuild Reserve for a four (4) node cluster would be 25% while it would be just 8% for a twelve (12) node cluster. This avoids imposing a generic 30% capacity overhead of a vSAN Cluster, thus dynamically optimizing reserved storage capacity on vSAN. - Recommendation is to design for vSphere cluster host counts large enough to lower the percentage needed for Reserved Capacity, This will provide the most efficient, yet agile vSphere cluster design. | - When enabled vSAN will impose two soft limits: - Operations Reserve - limits for capacity needed to perform transient storage activities like storage policy changes, re-balancing. - Host Rebuild Reserve - will reserve the capacity needed to absorb a sustained failure of a single ESX host in a vSAN cluster – to support an N+1 cluster design strategy. - "Host Rebuild Reserve" has the following implications in a vSAN storage cluster that consist of only 4 hosts. When paired with the Auto-Policy Management feature, this will prevent vSAN storage clusters from being able to use space-efficient and more performant RAID-5 policy. Caution vSAN capacity reserve is not supported in conjunction with the following vSAN Cluster typologies: - Three (3) node clusters. - vSAN stretched clusters. - vSAN clusters using fault domains. |
| VCF-VSAN-RCMD-CFG-004 | Enable disk Automatic Re-balance. | - When enabled vSAN will attempt to automatically keep the storage consumption of each capacity drive within a default variance threshold of 30%. - This allows vSAN to more evenly distribute the data across the discrete devices to achieve a balanced distribution of resources, and thus, improved performance. | - Adding or scaling out new hosts will trigger a disk rebalance and resynchronization. - When adding or replacing failed resources, automatic rebalance may introduce additional data re-synchronization and as a result additional network traffic utilization - re-synchronization may have an impact when vSAN clusters are network bandwidth constrained. |

Table 3-28. Common vSAN Network Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-NET-REQD-CFG-001 | vSAN requires a high speed, low latency network for synchronous replication of vSAN components. Note vSAN can use 10GbE , 25GbE or greater. | 10GbE is the minimum network required for vSAN (ESA and OSA architectures Caution There is limited support for 10GbE and vSAN ESA. For vSAN ESA ready node, vSAN-ESA-AF-0 profile, only supports 10GbE. Please refer to https://partnerweb.vmware.com/comp_guide2/vsanesa_profile.php | - Using 10GbE for vSAN networking can severely limit vSAN performance and can introduce bandwidth constraints combined with high latency. - Larger vSAN ESA nodes require 25GbE. Caution Using 10GbE networking on vSAN ESA will trigger a vSAN cluster compliance check. Please review Broadcom Knowledge base KB372309 |
| VCF-VSAN-NET-REQD-CFG-002 | - Use a dedicated VLAN for a single rack cluster Note You can share a single VLAN for multiple vSAN clusters within the same rack - For stretched cluster topology, use a dedicated VLAN per availability zone. See vSAN Stretched HCI Cluster Storage Model - For vSAN HCI multi-rack topology, VLAN requirements will depend on vSphere Multi-Rack Cluster Detailed Design if using L2 or L3 design | - For a single rack cluster using a dedicated VLAN allows for better storage traffic isolation and security. - For vSAN Stretched Cluster VCF automated workflows, it is required to have two (2) VLANs, one VLAN per availability zone. | - For a single rack Cluster a VLAN ID must be allocated for use for a vSAN Cluster. - For a vSAN Stretch Cluster topology two VLAN IDs must be allocated for use in a stretched cluster topology. These VLAN must be routable to each other - For vSAN HCI multi-rack topology, VLAN requirements will depend on the deployment, if using L2 or L3 design |
| VCF-VSAN-NET-REQD-CFG-003 | - Use one (1) IPv4 IP address subnet for vSAN traffic in a single rack cluster. - For vSAN stretch cluster topology, use two (2) IPv4 subnets, see VLAN and IP Subnet Requirements for vSAN Stretched Cluster in a VCF Environment | The vSAN VMkernel in a VMware Cloud Foundation platform requires IPv4 addressing. | - You require an IPv4 subnet configured with a correctly sized subnet to support the number of free IP addresses for each vSAN node in a single rack vSAN Cluster. - A gateway IP is required. |

Table 3-29. Common vSAN Network Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-NET-RCMD-CFG-001 | 25GbE or higher for vSAN Data traffic is recommended. | - vSAN synchronous replication requires high bandwidth / low latency intra-node communication. While 10GbE is supported for vSAN 25GbE or higher networking is highly recommended to ensure consistent performance. | None. |
| VCF-VSAN-NET-RCMD-CFG-002 | Small scale workloads: Recommend two (2) physical NICs per vSAN node for small scale workloads, that utilize smaller vSAN ready node profiles. For vSAN ESA ready node sizing guidance review https://partnerweb.vmware.com/comp_guide2/vsanesa_profile.php | - Simplifies deployment to use a single vDS model - During periods of contention NIOC will distribute shares among competing traffic | Requires careful planning to ensure vSAN workload can share resources with other services such as vMotion and NSX workloads. For high IO intensive workloads, 100GbE networking maybe required for vSAN ready nodes that require two phyiscal nics. |
| VCF-VSAN-NET-RCMD-CFG-003 | Large scale workloads: Recommend four (4) physical NICs per vSAN node For large scale / IO intensive workloads based on bigger vSAN ready node profiles. For vSAN ESA ready node sizing guidance review https://partnerweb.vmware.com/comp_guide2/vsanesa_profile.php | - This guarantees storage traffic isolation and full bandwidth of physical network resources on a ESX host. | Additional vSphere Distributed Switches requires increasing management overhead with a higher number of ESX host physical NICs. For more information see Storage and Workload Separation Network Model For high IO intensive workloads, 100GbE networking maybe required to satisfy bandwidth and latency requirements. |
| VCF-VSAN-NET-RCMD-CFG-004 | Configure the MTU size of the vSphere Distributed Switch to 9000 bytes for jumbo frames. See VCF-VDS-RCMD-CFG-003 | - Supports the MTU size required by system traffic types. Improves traffic throughput. | When adjusting the MTU packet size, you must also configure the entire network path (VMkernel ports, virtual switches, physical switches, and routers) to support the same MTU packet size. |
| VCF-VSAN-NET-RCMD-CFG-005 | For vSphere Single-Rack Cluster Model use Layer 2 networking for vSAN. | Layer 2 connectivity between all vSAN hosts sharing the subnet within a single rack simplifies deployment. | None. |
| VCF-VSAN-NET-RCMD-CFG-006 | For Layer 3 Multi-Rack Cluster Model with Layer 3 networking for vSAN. | Reduces fault domain of network to a specific rack and IP segment. | - Routing must be configured between each network subnet. - Host to rack addressing needs to be planned in advance. - Consider the number of hops and additional latency incurred while the traffic gets routed. |
| VCF-VSAN-NET-RCMD-CFG-007 | Use the Failover Order teaming algorithm for the vSAN storage port group. See VCF-VDS-SS-RCMD-DPG-001 | Provides a consistent traffic flow through a single physical ToR switch without need to traverse Inter Switch Link during normal operations. | It needs to be manually configured during deployment using vSphere Distributed Switches custom profile option. |

Table 3-30. vSAN ESA Storage Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-ESA-REQD-CFG-001 | Ensure vSAN certified Drives used for vSAN ESA clusters. | - When using VMware Cloud Foundation automated workflows, vSAN ESA cluster creation maybe blocked unless vSAN certified drives are detected. - vSAN Auto policy will be enabled when vSAN certified components are detected. This allows vSAN to generate an optimal default storage policy, based on the cluster topology and the number of ESX hosts. | None. |
| VCF-VSAN-ESA-REQD-CFG-002 | 1.6 TB drive size or higher for vSAN ESA devices for storage pools. | Minimum supported vSAN ESA drive size is 1.6TB. | vSAN ESA supports up to 24 drives per node. Smaller capacity drives may constrain raw capacity of a vSAN node. |
| VCF-VSAN-ESA-REQD-CFG-003 | 16 Cores or higher per vSAN node. vSAN ESA has a minimum value of 16 cores per vSAN Node | Minimum vSAN ESA core count per node is 16 for the smallest vSAN Ready node, vSAN-ESA-AF-0. | Smaller number of cores per node may require more vSAN nodes to support targeted workloads. |
| VCF-VSAN-ESA-REQD-CFG-004 | Memory size per vSAN Ready node 128GB or higher | Minimum vSAN ESA memory is 128GB for the smallest vSAN Ready node. vSAN-ESA-AF-0. | - Smaller memory footprint per node will limit number of vSAN drives per node. - Smaller memory footprint per node for vSAN ESA will limit VM workloads being placed on vSAN nodes. |

Table 3-31. vSAN ESA Storage Model Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-VSAN-ESA-RCMD-CFG-001 | - Specific size of drives used will depend on target workload. - The recommendation is to use multiple storage devices per vSAN node - NVMe TLC devices with a performance Class F (100000 - 349999) or higher with endurance class 3 DWPD or higher. | - The number of storage devices has an impact on the performance of vSAN ESA. - NVMe drives with mixed-use endurance and performance class typically classified on the vSAN HCL as "3DWPD" (3 Drive Writes per Day) will give a balance between performance and durability. | - Smaller drive sizes will impact overall capacity of vSAN ESA. Large drive sizes will introduce larger capacity fault domain in case of failure. - Sizing vSAN Nodes requires determining the workload capacity requirements prior to deployment. Refer to vSAN Sizer for recommended capacity at https://vcf.broadcom.com/tools/vsansizer/home. |
| VCF-VSAN-ESA-RCMD-CFG-002 | Its recommended to use vSAN ready nodes with high core count. | vSAN Express Storage Architecture (ESA) is designed to be highly parallel and leverages multiple CPU cores for improved performance, making it multithreaded, and optimized for NVMe- TLC based flash devices. | There will be a correlation between target workload, number of cores and NVMe devices per vSAN Node. Refer to vSAN Sizer for recommended capacity at https://vcf.broadcom.com/tools/vsansizer/home. |
| VCF-VSAN-ESA-RCMD-CFG-003 | Its recommended to use vSAN ready nodes with high memory capacity per vSAN node. | When enabled vSAN has required memory overheads and must have adequate resources account for VM workloads. | Refer to vSAN Sizer for recommended capacity at https://vcf.broadcom.com/tools/vsansizer/home. |
| VCF-VSAN-ESA-RCMD-CFG-004 | Recommend at least four (4) nodes for vSAN cluster size. | This ensures in the event of a fault domain failure, vSAN objects can be re-protected to maintain policy compliance. In conjunction with auto policy management, four (4) node vSAN ESA clusters will allow vSAN to automatically create a default storage policy that is capacity optimized. | None. Note vSAN Auto Policy default profile can be can be adjusted by an admin if necessary. |
| VCF-VSAN-ESA-RCMD-CFG-005 | - It is recommended to select the same minimum requirements for vSAN Storage cluster nodes as vSAN ESA HCI nodes. - Please review vSAN Storage minimum node requirements | - vSAN Storage Cluster Nodes use the same architecture as regular vSAN ESA nodes. - vSAN ESA nodes can be re-purposed for vSAN Storage Clusters or vSAN HCI Clusters | None. |

#### Network Consumption Models

VMware Cloud Foundation (VCF) offers various approaches to network consumption. Administrators can select the model the best suit their requirements regarding how network and security services are created and life-cycled.

VCF Automation All Apps Orgs network consumption model

When the goal is to leverage the VCF infrastructure via VCF Automation via All Apps Orgs, where the management of compute , storage and and network is centralized via single UI/API, the cloud user should not interact directly with the NSX UI/API other than for troubleshooting purposes. The cloud user should create and administer NSX VPCs via the VCF Automation API/UI.

Network Consumption models based on custom topologies

This category encompasses designs where the deployment of workloads and network topologies are decoupled or only loosely coupled. A common example is a scenario where the network administrator manually creates a topology for a specific use case or tenants in NSX, then the VI Admin deploys workloads on top of that manually or via some automation.

Leveraging automation implies that the automation solution can discover the NSX-created networks (e.g., VPC subnets) or the corresponding vSphere dvpgs so that workloads can be connected to them. The automation solution doesn't require any awareness of how those networks are wired into the larger network design, which can be independently controlled by the network administrator in NSX. The network administrators have full freedom to deploy the Network Workload Design that best meets the application requirements. Various Network Workload designs are available and presented in the relevant section. The NSX administrator can select one for the entire infrastructure or mix multiple designs of the same or different types for different tenants or use cases.

The network consumption models based on custom topologies we will cover are:

- Virtual Private Cloud (VPC) Network Virtualization model

- NSX Segment Network Virtualization model

- VLAN Networking model

Virtual Private Cloud (VPC) Network Virtualization model

A VPC is a cloud networking construct, a secure, isolated virtual network within a cloud environment. NSX VPCs bring the same characteristics and consumption model of the public cloud to VCF.

The NSX VPC is a foundational and required component for designs requiring VCF Automation All Apps Orgs, but it is also the recommended network consumption model for any custom topology aimed at supporting the manual deployment of workloads, VCF Automation VM Only Orgs, or third-party automation tools.

Starting with VCF 9, VPCs can be managed from the vCenter inventory networking tab making the vSphere client the single interface for managing compute, network, and storage for simple single-tenant deployments. Multi-tenant deployments not requiring the strong separation provided by VCF Automation All Apps Orgs can leverage VPCs within the NSX multi-tenancy framework to provide self-service network and security services via the NSX UI and API.

NSX Segment Network Virtualization model

This model leverages Policy NSX objects such as Tier-0/Tier-1 Gateways and segments to deploy network topologies. This model allows the Network Administrator to leverage all of the advanced NSX network capabilities but requires more extensive networking and NSX-specific knowledge.

This model does not support VCF Automation All Apps Orgs, and while it does support supervisor services, it is recommended that new deployments of supervisor embrace the VPC-based topology.

For custom topologies, the VPC model is still the preferred approach for its simplicity and seamless integration with other VCF components such as vCenter.

The NSX Segment Network Virtualization model should be considered when the capabilities it provides compared to the VPC model are critical to meeting the design requirements. Among those, the ability to extend network topologies across workload domains in different VCF instances via NSX Federation is a primary use case.

Table 3-32. Comparison between NSX Segment and VPC models

| Exclusive capabilities of the NSX Segment Network Virtualization Model | Alternatives when using the VPC Network Virtualization model |
| --- | --- |
| NSX Federation: - Stretched Layer 2 and Layer 3 topologies across NSX Domains - Single point of management for security policies across NSX Domains - VM Mobility across NSX Domains | - Use a shared NSX Domain across VI WLDs - With separate NSX Domains, the NSX Enterprise Admin can leverage NSX federation to provide a single point of management for security policies across VPCs in different NSX domains - Duplicating VPCs with the same networking layout across NSX Domains allows the recovery of VMs one VPC at a time (It requires filtering the VPC IP ranges at the secondary site on the upstream Tier-0 gateway or VRF until the recovery process is initiated) |
| VPNs on Tier-1 Gateways | Configure VPNs on upstream VRF or Tier-0 Gateway, where dynamic routing is supported and peer redundancy can be provided. |
| Advanced Security Features supported on Tier-1 Gateway (Firewall Add-On required): - Identity Firewall - URL Filtering - FQDN Analysis - IDS/IPS - Malware Detection - TLS Inspection | - Enable those capabilities on the upstream VRF or Tier-0 Gateway when supported(i.e., IDS/IPS, Identity Firewall), Firewall Add-On is required - Enable the equivalent Distributed Service when available (i.e. Distributed IDS/IPS or Distributed Malware Protection), Firewall Add-On is required |

VLAN Networking Model

In this model, the only VCF component participating in the networking functionalities is the VDS, responsible for switching the traffic between the VMs running in the hypervisor and the physical uplinks. Once the packets are delivered to the upstream physical switch, it is the responsibility of the physical fabric to deliver them to the appropriate destination.Virtual machines are connected to distributed port-groups or NSX VLAN segments.

This model lacks the benefits provided by the NSX network virtualization which decouples the physical network from the topology requirements of the applications and services that run on top of it. With network virtualization, the physical fabric can be designed and operated based on a simpler and more stable model that does not require following the application's lifecycle. The role of adapting to the ever-changing application requirements is offloaded to the NSX network virtualization layer while the physical fabric design can be centered around maximizing throughput and high availability, minimizing latency, and optimizing operational efficiency.

The VLAN networking model is supported in VCF and may be appropriate for specific use cases, but it is not recommended for the majority of scenarios as it lacks the benefits provided by network virtualization. The benefits of network virtualization are summarized in the table below.

Comparison VLAN Networking vs Network Virtualization (VPC or NSX Segment)

| Attribute | VLAN Networking | Network Virtualization (VPC or Policy NSX) |
| --- | --- | --- |
| VM Mobility | VM mobility is limited to the span of a VLAN (usually one rack) | VM mobility across Layer 3 boundaries – any topology |
| Rack Availability | Achieving rack availability via vSphere HA is not possible without extending VLANs between racks | Easy to achieve rack availability via vSphere HA for compute workloads regardless of the network fabric topology |
| Resource Pooling | Resource pooling is limited by the VLAN span | Heterogenous islands of computing are easily integrated. vSphere clusters can be distributed across N racks regardless of the network fabric topology |
| Infrastructure Agility | Slow application deployment dependent on physical network reconfigurations | The physical network is abstracted. No changes are required to support the application lifecycle |
| Change Management | Network changes require the reconfiguration of multiple devices | Single point of management for the entire logical network. The network can embrace an Infrastructure as Code (IoC) approach, where network constructs follow the application lifecycle |
| Disaster Recovery | Disaster recovery requires re-IP or extending networks across sites via the physical network | Easy disaster recovery without re-IP via NSX Federation (NSX Segment only) or Multi-AZ design. |
| L2 Scale | Fundamentally tied to MAC learning and L2 discovery over different types of underlay technology (e.g., traditional L2, EVPN) | Programmed from vSphere, and thus no MAC learning and abstracted from the physical network, reducing the leaf switch scale requirements |
| L2 Security | Physical fabric is directly exposed to L2 frames generated by the VMs (e.g., STP BPDU) | NSX Overlay isolates the physical fabric from the Layer 2 frames generated by the endpoints |
| Support of Advanced Networking (i.e., NAT, Firewall) | Tied to physical ASIC and limited by switch vendor | Software-defined allows scaling of the networking and security stack |

When using VLAN networking two options are available:

- Create distributed port-groups (dvpgs) in vCenter

- Create VLAN Segments in NSX Manager

When enabling "NSX on dvgps" for a vSphere cluster in NSX, workloads connected to dvpgs can leverage the same NSX distributed services as those connected to NSX Segments or VPC subnets. Those services include the vDefend Distributed Firewall (available as an advanced service), spoofguard, and any other services that can be applied via NSX segment profiles.

To use VLAN segments, workloads must be placed on the NSX-managed distributed virtual port groups that are automatically created when a VLAN segment is defined in NSX.

From a scalability perspective, using VLAN segments is better suited for larger environments with a high number of VDSs, vCenters, and dvpgs. When creating a VLAN segment in NSX, a corresponding vCenter dpvg is automatically created on any VDS part of the corresponding VLAN TZ. This provides simpler lifecycle management of VLAN networks via NSX manager in environments with numerous VDSs, across single or multiple vCenters. This consideration is particularly relevant for VMware Cloud Foundation platforms where each vSphere cluster has a dedicated VDS, increasing the number of VDSs and dvpgs to be managed.

Table 3-33. Comparison Distributed Switch Port-Groups vs VLAN Segments

| Feature | Option 1: vCenter Distributed Switch Port-Groups | Option 2: NSX VLAN Segments |
| --- | --- | --- |
| Creation Method | Created directly in vCenter | Created in NSX Manager |
| NSX Services Support | Supported via "NSX on dvpgs" feature | Natively supported |
| Compatible Services | Distributed firewall, spoofguard, segment profiles (They require applying NSX to distributed port-groups) | Distributed firewall, spoofguard, segment profiles |
| Workload Placement | VMs connected to vCenter-managed dvpgs | VMs connected to NSX-managed dvpgs (auto-created) |
| Scalability | Suitable for smaller environments | Better for larger environments with many VDSs and vCenters |
| Lifecycle Management | Individual management of dvpgs in vCenter | Simplified management through NSX Manager |
| Multi-VDS Deployment | Requires manual creation of dvpgs on each VDS | Auto-creates corresponding dvpgs on all VDSs in VLAN TZ |

NSX Federation

NSX Federation provides a single point of management for workload networking and security across NSX domains, regardless if those NSX domains are associated to workload domains, within, or across VCF Instances.

NSX Federation is compatible with all the network consumption models listed in the previous section but the level of integration and the benefits it can provide vary across them. The table below summarize its capabilities in the context of each of the available network consumption models.

The capabilities considered are :

- NSX Enterprise Admin centrally manages security policy and groups across NSX Domains

- Layer 2 and Layer 3 network topology extension across NSX Domains

Note	Tenant (NSX Project) admins, and VPC admins do not have the ability to configure global groups and security policies spanning multiple NSX Domains, nor managing local groups and security policies for multiple NSX Domains via the NSX Global Manager.

Table 3-34. NSX Federation capabilities in the different Network Consumption Models

| Network Consumption Model |  |  |
| --- | --- | --- |
|  | NSX Enterprise Admin centrally manages security policy and groups across NSX Domains | Layer 2 and Layer 3 network topology extension across NSX Domains |
| VCF Automation All Apps Orgs | Yes. Tenant or VPC scoped groups and security policies are local to a single NSX Domain. | No |
| Virtual Private Cloud (VPC) Network Virtualization | Yes. Tenant or VPC scoped groups and security policies are local to a single NSX Domain. | No |
| NSX Segment Network Virtualization | Yes | Yes |
| VLAN Networking | Yes | No |

When extending networking and security services across workload domains part of the same VCF Instance, the goal can be achieved via NSX Federation or by sharing the NSX domain across the workload domains. When extending networking and security services across workload domains as part of different VCF Instances, NSX Federation is the only option. For workload domains that are part of the same VCF Instance, the two methods are compared in the table below

Table 3-35. Comparison of NSX Federation vs Shared NSX Domain Capabilities for cross Workload Domain Networking

| Attribute | NSX Federation | Shared NSX domain | Considerations |
| --- | --- | --- | --- |
| Cross workload domain traffic | Cross workload domain traffic goes trough the NSX Edge nodes. | Cross workload domain traffic leverages fully distributed forwarding on the ESX host. | Consider the amount of cross workload domain traffic. If NSX Federation is selected, size the NSX Edge node accordingly, and distribute workloads on separate segments and Tier-1 gateways to spread the load across multiple NSX Edge nodes. |
| MTU | Cross workload domain traffic supports any MTU via TCP mss adjustment and fragmentation. | Appropriate MTU must be available end-to-end (workload MTU + 200 bytes). | If MTU cannot be guaranteed end-to-end, NSX Federation provides a solution. |
| Local egress | Yes | No | Implementing Local egress causes asymmetric north-south routing across the workload domains unless each stretched segment has workloads in a single workload domain and inbound routing is configured appropriately. |
| Management plane availability | Active/Standby model for Global Managers. Active and Standby clusters can be deployed in the same or different VCF Instances. Layer 3 connectivity is required. No layer2 connectivity is required between Active and Standby Global Manager cluster. | Single NSX Manager cluster across workload domain. Recovery must be provided by the underlying infrastructure, for example via a stretched vSphere cluster. | NSX Federation is the preferred solution for disaster recovery use cases. |
| Control plane availability | Independent per workload domain. | Shared across workload domains. | In a design with a shared NSX domain accross workload domains model requiring the recovery of workloads across such workload domains, workloads cannot be recovered until the control plane (NSX Manager appliances) is recovered. This limitation does not affect designs based on NSX Federation as each workload domain has an independent local control plane. |
| Feature support | Some NSX features are not available. Refer to the NSX documentation for an up to date comprehensive list. | Full NSX feature support. | When the design requires the adoption of specific capabilities, NSX Federation may not meet those requirements. |

#### Simple NSX Manager Model

This model addresses use cases requiring a minimal infrastructure footprint, lower scale, and lower availability requirements.

Simple NSX Manager Model Attributes

| Attribute | Detail |
| --- | --- |
| Deployment Size | Single NSX Manager Appliance with Internal VIP |
| Availability | - vSphere HA protects the node applying high restart priority - Slower recovery from node failure |
| Infrastructure Impact during Upgrade | - Management and control plane impacted during upgrade operations. - Deployment/Power-on/Restart/vMotion of VMs may be impacted. |
| Virtual Networking Scale | Reduced scale |
| API Request Scale | Limited scale |

Table 3-36. Common NSX Manager Design Requirements for All Models

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-LM-REQD-CFG-001 | Place the appliances of the NSX Manager cluster on the VM management network in the management domain. | - Simplifies IP addressing for management compomentes by using the same VLAN and subnet. - Provides simplified secure access to management components in the same VLAN network. | None. |
| VCF-NSX-LM-REQD-CFG-002 | Create a virtual IP (VIP) address for the NSX Manager cluster for the workload domain. | - Utilized for user interface and API of NSX Manager. - In a clustered configuration, provides high availability. - In a non-clustered configuration, prepares for future expansion. | - In a clustered configuration, the VIP address feature provides high availability only. It does not load-balance requests across the cluster. - When using the VIP address feature, all NSX Manager nodes must be deployed on the same Layer 2 network. |
| VCF-NSX-LM-REQD-CFG-003 | In vSphere HA, set the restart priority policy for each NSX Manager appliance to high. | - NSX Manager implements the control plane for virtual network segments. vSphere HA restarts the NSX Manager appliances first so that other virtual machines that are being powered on or migrated by using vSphere vMotion while the control plane is offline lose connectivity only until the control plane quorum is re-established. - Setting the restart priority to high reserves the highest priority for flexibility for adding services that must be started before NSX Manager. | If the restart priority for another management appliance is set to highest, the connectivity delay for management appliances will be longer. |

Table 3-37. Common NSX Manager Model Design Recommendations for All Models

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-LM-RCMD-CFG-001 | Deploy appropriately sized nodes in the NSX Manager cluster for the workload domain. Note Check VMware Configuration Maximums to select the right NSX Managers form factor for your scale needs. | Ensures resource availability and usage efficiency per workload domain. | You must have sufficient resources in the management domain default vSphere cluster to run three (3) NSX Manager nodes. |

Table 3-38. NSX Manager Simple Deployment Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-LM-REQD-CFG-004 | Deploy a single NSX Manager node in the management domain default vSphere cluster for configuring and managing the network services for the VCF domain. | Supports the objective of a small scale deployment. | - Slower recovery from node failure. - Management and control plane impacted during upgrade operations. - Supports limited scale. |

Table 3-39. Common NSX Manager Design Recommendations for Stretched Clusters for All Models

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-LM-RCMD-CFG-002 | Add the NSX Manager appliances to the virtual machine group for the first availability zone. | Ensures that, by default, the NSX Manager appliances are powered on a host in the primary availability zone. | None. |

#### Host Fault Tolerant NSX Edge Cluster Model

A vSphere Single-Rack Cluster Model for hosting NSX Edge nodes offers a minimalist footprint while maintaining basic host-level availability.

Figure 3-8. Host Fault Tolerant NSX Edge Cluster Model

Host Fault Tolerant NSX Edge Cluster Model Attributes

The Host Fault Tolerant NSX Edge Cluster Model has the following attributes.

| Attribute | Details |
| --- | --- |
| Availability model | - Host level redundancy only. - Single shared storage. - This design cannot ensure continuous service availability for north-south communication and centralized network services during a complete rack failure. |
| NSX Edge node mobility across racks | N/A |
| BGP peering | Establish BGP peering solely with the local ToR switches located within the same rack that hosts the NSX Edge node. |
| NSX Edge node and link failover mechanisms | - Fast NSX Edge node High-Availability detection for NSX Edge node failures. - Fast link failure detection with Bidirectional Forwarding Detection (BFD) for BGP. |
| North-South bandwidth growth | - Add hosts to the single existing vSphere cluster or preferably transition to a rack fault tolerant design with two or more vertical Layer 2 vSphere cluster in separate rack. - When deploying more NSX Edge nodes on existing hosts, ensure each host has sufficient CPU, memory, and importantly, additional physical NICs (pNICs) to maintain maximum packet forwarding rates (PPS) for each NSX Edge node. |

Table 3-40. Common NSX Edge for Single Availability Zone Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-EDGE-REQD-COMSAZ-01 | Align the Tunnel Endpoint (TEP) MTU for NSX Edge nodes to the same MTU as the VDS MTU. | Consistent MTU within the same NSX Overlay Transport Zone is required. | Requires manual configuration of the Tunnel Endpoint (TEP) MTU value in NSX Global Fabric Settings. |

Table 3-41. Common NSX Edge for Single Availability Zone Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-EDGE-RCMD-COMSAZ-01 | Dedicated vSphere cluster to exclusively host NSX Edge nodes. | - NSX Edge nodes use dedicated and exclusively host resources (CPU, memory, pNIC) to provide best packet forwarding performance, reliability and operational simplification. - Enables the selection of an independent vSphere Distributed Switch design separate from the vSphere cluster hosting workload virtual machines. - Enable specific hardware selection for NSX Edge node hosts. | Additional servers are required. |
| VCF-NSX-EDGE-RCMD-COMSAZ-02 | Limit the number of NSX Edge nodes per host to the number of available physical NICs. | Each NSX Edge node should be connected to a pair of physical NICs to ensure maximum packet forwarding rates (PPS). | Additional physical NICs or hosts, or both are required to support the deployment of a higher number of NSX Edge nodes. |
| VCF-NSX-EDGE-RCMD-COMSAZ-03 | Use the default memory reservation with 100% for NSX Edge nodes. | The 100% memory reservation configuration is a guaranteed minimum allocation of host physical memory that is entirely dedicated to a NSX Edge node, ensuring this full memory amount remains available to the NSX Edge node even when the host system is in an over-committed state. | In a collapsed cluster where other workload VMs have memory reservation too: - Virtual machines may fail to power on when memory reservations can't be guaranteed, even if idle reserved memory exists. - vSphere HA may be unable to restart VMs after host failures if it cannot meet their memory reservations on surviving hosts.Memory reservation locks physical RAM, while CPU reservation guarantees minimum access to a shared resource. |
| VCF-NSX-EDGE-RCMD-COMSAZ-04 | Use the default CPU reservation priority in NSX set to high shares for NSX Edge nodes. | Setting CPU shares to "High" ensures that NSX Edge nodes receive higher relative priority during CPU contention scenarios, allowing critical network functions to maintain performance when host resources are constrained. | High CPU reservations are generally less critical than high memory reservations. |
| VCF-NSX-EDGE-RCMD-COMSAZ-05 | For each NSX Edge node, assign a dedicated physical NIC to the data plane fastpath interfaces fp-eth0 and fp-eth1. | - Each NSX Edge node leverages only two of four data plane fastpath interfaces. - When two NSX Edge nodes are deployed per host, each NSX Edge node should use a dedicated pair of physical NICs - one active and one standby. - When deploying two NSX Edge nodes per host, each NSX Edge node requires two dedicated physical NICs configured for optimal performance. | The two remaining data plane fastpath interfaces per NSX Edge node are not used. |
| VCF-NSX-EDGE-RCMD-COMSAZ-06 | Map data plane fastpath interface fp-eth0 with physical NIC 1 as active and physical NIC 2 as standby; map data plane fastpath interface fp-eth1 with physical NIC 1 as standby and physical NIC 2 as active, creating complementary failover paths. | Enables redundant paths for both data plane fastpath interfaces on each NSX Edge node, preventing single points of failure. | None. |
| VCF-NSX-EDGE-RCMD-COMSAZ-07 | Create a new dvPortGroup for the NSX Edge node management interface, separate from the dvPortGroup used for ESX host management VMkernel. Depending on the selected NSX Edge cluster design, an NSX Edge node management dvPortGroup is required for each vSphere cluster, rack, or Availability Zone. | Traffic segregation enables organizations to implement and enforce network security policies. | - Increased configuration effort. - For a management domain: a separate dvPortGroup for the NSX Edge node management interface already exists when using a single VDS design; this is the same dvPortGroup as used for the management stack. |
| VCF-NSX-EDGE-RCMD-COMSAZ-08 | Create for every host an individual host group. | Host groups will be required to enable compute policy type Best Effort Restart (BER). | Host groups must be configured through the administrator before initiating the NSX or vCenter Network Connectivity workflow. |
| VCF-NSX-EDGE-RCMD-COMSAZ-09 | Create for every NSX Edge node an individual VM group. | VM groups will be required to enable compute policy type Best Effort Restart (BER). | The NSX or vCenter Network Connectivity workflow automatically configures VM groups when the host group affinity option is selected. Do not create this VM group manually. |
| VCF-NSX-EDGE-RCMD-COMSAZ-10 | Assign to each individual NSX Edge node the previously configured host group. | Each NSX Edge node has a 1:1 affinity with the underlying ESX host. | If the assignment was not set through the NSX or vCenter Network Connectivity workflow , it can be added via NSX API. |
| VCF-NSX-EDGE-RCMD-COMSAZ-11 | Create a VM/Host "must" rule by mapping the previously created host group and VM group together, ensuring the NSX Edge node in the VM group must only run on this specific host. | VM/Host "must" rule for each NSX Edge node will be required to enable compute policy type Best Effort Restart (BER). | The NSX or vCenter Network Connectivity workflow automatically configures VM/Host rule when the host group affinity option is selected. Do not create this VM/host rule manually. |
| VCF-NSX-EDGE-RCMD-COMSAZ-12 | Create a vCenter tag with a category to identify the compute policy type Best Effort Restart (BER) and assign it to each NSX Edge node. | The vCenter tag with a category is required to map both the compute policy type Best Effort Restart (BER) and the NSX Edge nodes together. | The NSX or vCenter Network Connectivity workflow automatically configures the vCenter tag with a category and assign to each NSX Edge node when the host group affinity option is selected. Do not create the vCenter tag and category manually. |
| VCF-NSX-EDGE-RCMD-COMSAZ-13 | Create a compute policy type Best Effort Restart (BER) and assign the previously created vCenter tag and category to it. | - Compute policy type Best Effort Restart (BER) enables smooth rolling host upgrades leveraging graceful NSX Edge node shutdown to minimize traffic disruption. - The compute policy type Best Effort Restart (BER) verifies peer NSX Edge node readiness during host upgrades through multiple pre-checks before initiating NSX Edge node failover and ensures the NSX Edge node shutdown process begins gracefully only after successful pre-checks. | - The NSX or vCenter Network Connectivity workflow automatically configures the compute policy type Best Effort Restart (BER) when the host group affinity option is selected. Do not create this compute policy type Best Effort Restart (BER) manually. - Evacuating the NSX Edge node using vMotion during host upgrades or maintenance is not possible. - User-initiated host maintenance mode will shut down the hosted NSX Edge node regardless of whether a healthy peer NSX Edge node is available to take over, as readiness pre-checks are not executed during this process; it is the administrator's responsibility to ensure service availability is maintained. |
| VCF-NSX-EDGE-RCMD-COMSAZ-14 | Select the NSX Edge node size large or x-large. | - The size of an NSX Edge node depends on the throughput requirements and any configured centralized network services. - For most deployments, particularly when NSX Edge nodes are used solely for north-south traffic forwarding, the large NSX Edge node size provides an optimal balance between performance and resource utilization. | - You must provide sufficient compute resources to support the chosen NSX Edge node size. - In vSphere clusters with a high density of NSX Edge nodes, such as multi-tenancy environments with high oversubscription ratios or collapsed vSphere clusters where both workload virtual machines and NSX Edge nodes run on the same hosts, the resource requirements of large or extra large NSX Edge nodes occasionally create resource constraints. |
| VCF-NSX-EDGE-RCMD-COMSAZ-15 | Select the auto-generation password option for NSX Edge nodes. | Auto-generation password option enables centralized password management in VCF Operations. | Password needs to be looked up to gain SSH access. |
| VCF-NSX-EDGE-RCMD-COMSAZ-16 | Select static IP allocation for the NSX Edge node management interface. | Static IP allocation removes the dependency on external DHCP services. | None. |
| VCF-NSX-EDGE-RCMD-COMSAZ-17 | - Configure separate Tunnel Endpoint (TEP) VLANs for NSX Edge nodes and ESX hosts. - It is possible to use a single TEP VLAN across host and NSX Edge node by enabling "NSX on dvPG". | - Simplifies Tunnel Endpoint (TEP) IP management by separating IP allocation methods between ESX hosts and NSX Edge nodes, for example using static IP assignment for NSX Edge nodes while utilizing DHCP-based allocation for ESXi hosts. - No need to enable "NSX on dvPG". | - NSX Edge nodes and host Tunnel Endpoint (TEP) require separate VLANs with routing configured between them to enable proper overlay network communication. - Requires the configuration of ab additional VLAN for NSX Edge node Tunnel Endpoint (TEP) traffic. |
| VCF-NSX-EDGE-RCMD-COMSAZ-18 | Configure IP Pool allocation for the NSX Edge node Tunnel Endpoint (TEP) interfaces. | - IP Pool allocation eliminates dependency on external DHCP services while reducing configuration overhead compared to static IP assignment. - Each of the two data plane fastpath interfaces get an IP address assigned to enable NSX Edge node Tunnel Endpoint (TEP) | Requires the configuration of one or more IP Pools depending on the vSphere cluster model. |
| VCF-NSX-EDGE-RCMD-COMSAZ-19 | Enable TEP Groups for NSX Edge nodes. | - Higher throughput achievable by a single NSX Edge node. - Granular flow base load balancing of NSX Edge node TEP traffic. - Resiliency to partial link failures (Physical link state up but not forwarding). | TEP Groups must be enabled through NSX API globally for all NSX Edge nodes. |
| VCF-NSX-EDGE-RCMD-COMSAZ-20 | Limit the span of NSX Edge node networks to the local racks. | - VLANs including NSX Edge node Tunnel Endpoint (TEP), the NSX Edge node management and the two (2) Edge Uplink VLANs that carry BGP sessions and north-south traffic are limited to the rack. - BGP will establish peering connections exclusively with Top-of-Rack (ToR) switches located within the same rack, ensuring a deterministic alignment between physical components (ToRs and cabling) and the corresponding BGP session. | - Limit the span of networks to reduce failure propagation across racks. - Avoid BGP peering across racks. |
| VCF-NSX-EDGE-RCMD-COMSAZ-21 | Use a single Edge Switch in the NSX Edge nodes. | - Simplifies deployment of NSX Edge nodes. - Supports multiple Tunnel Endpoint (TEP) interfaces. - Carries both overlay and uplink traffic. | The NSX or vCenter Network Connectivity workflow automatically configures always a single Edge Switch. |

Host Fault Tolerant NSX Edge Cluster Model Design Requirements

This design requirement is a specific requirement in addition to the common design requirements.

Table 3-42. Host Fault Tolerant NSX Edge Cluster Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-EDGE-REQD-HFTSR-01 | Deploy two (2) or more NSX Edge nodes in the same vSphere Cluster. | - NSX Edge nodes are required to support centralized network connectivity for the virtual network. - At least two (2) NSX Edge nodes are required to support NSX Edge high-availability. | - Cluster resources must be available to support the NSX Edge nodes. - Additional NSX Edge nodes may be required to satisfy increased bandwidth requirements. |

Host Fault Tolerant NSX Edge Cluster Model Design Recommendations

These design recommendations are specific recommendations in addition to the common design recommendations.

Table 3-43. Host Fault Tolerant NSX Edge Cluster Model Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-NSX-EDGE-RCMD-HFTSR-01 | Ensure that NSX Edge nodes belonging to the same NSX Edge cluster are evenly deployed across different hosts.. | Maintain High-Availability and prevent single points of failure | Additional servers are required. |
| VCF-NSX-EDGE-RCMD-HFTSR-02 | Configure an NSX failure domain for each host where the NSX Edge node can run - this is only valid when you have more than two NSX Edge nodes; map the NSX Edge nodes to the NSX failure domains appropriately. | - Failure domains guarantee service availability in case of a failure affecting multiple NSX Edge nodes. - Active and standby instance of a VPC or Tier-1 gateway always run in different failure domains. | - Additional manual configuration. - This applies only for designs incorporating VPC or Tier-1 gateway in A/S mode. |

### VCF Operations Design

This section describes the design used for VCF Operations.

#### Simple VCF Operations Model

The Simple VCF Operations Model deploys a single node of all the VCF Operations components and uses vSphere HA as an availability mechanism.

The Simple VCF Operations Model includes three single-node components.

Figure 3-9. Simple VCF Operations Model

Simple VCF Operations Model Attributes

A Simple VCF Operations Model has the following attributes.

| Attribute | Detail |
| --- | --- |
| Availability | Relies on vSphere HA to restart the nodes in the event of an ESX host failure. |
| Scalability | Supports scale-up and scale-out. Note Scale-out is supported for the VCF Operations data nodes and VCF Operations collector nodes. |
| Recoverability | Restore through a backup. |

Simple VCF Operations Model Options

A Simple VCF Operations Model can leverage options from the design areas listed below, depending on objectives.

| Design Area | Options |
| --- | --- |
| Networking | - Default VM management distributed port group - Dedicated distributed port group - NSX segment (overlay or VLAN backed) |
| Licensing | - Connected Mode - Disconnected Mode |

Simple VCF Operations Model Design Requirements

Table 3-44. Simple VCF Operations Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-OPS-REQD-SIM-001 | Deploy VCF Operations as a single node in the default management vSphere cluster. | - Supports the simple deployment objective. - Provides the capacity required for monitoring virtual machines or objects within the monitoring scale limits. - Supports scale-out to a high availability model. | - You must select a size for the VCF Operations cluster nodes based on the known or forecasted virtual machine and object count across each connected VCF Instance to adhere to the scale limits. - No application level high availability. |

Table 3-45. Common VCF Operations Design Requirements for All Models

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-OPS-REQD-COM-001 | Deploy a single VCF Operations fleet management appliance in the default management vSphere cluster. | Provides life cycle management operations for VCF Operations fleet management components. | None. |
| VCF-OPS-REQD-COM-002 | Protect VCF Operations fleet management appliance by using vSphere HA. | Ensures availability of VCF Operations fleet management appliance without requiring manual intervention during an ESX host failure event. | None. |
| VCF-OPS-REQD-COM-003 | Deploy a single VCF Operations collector appliance in the default management vSphere cluster. | - Removes the load from the VCF Operations analytic node(s) from collecting metrics from local-instance applications. - Reuse VCF Operations collectors to gather both logs and metrics across the VCF environment. - Centralized management of logs and metrics, reducing administrative overhead. | You may need to scale-up the VCF Operations collector nodes to a greater size based on the known or forecast virtual machine and object count across each connected VCF Instance to adhere to the scale limits. |
| VCF-OPS-REQD-COM-004 | Protect VCF Operations collector appliance by using vSphere HA. | Ensures availability of VCF Operations collector appliance without requiring manual intervention during an ESX host failure event. | None. |

Simple VCF Operations Model Design Recommendations

Table 3-46. Common VCF Operations Design Recommendations for All Models

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-OPS-RECD-COM-001 | Enable data persistence on each VCF Operations collector appliance. | Provides the ability to store data in case of connectivity issues. | Storage availability on each VCF Operations collector must be monitored. |
| VCF-OPS-RECD-COM-002 | Configure VCF Operations to use a notifications configuration to route notifications for system events. | Integrates VCF Operations system events notifications to platform engineers, providing an enhanced user experience. | You must configure and maintain the notification integration. |
| VCF-OPS-RECD-COM-003 | Add a physical data center object to the VCF Operations configuration for the geographic location of each VCF Instance. | Provides organization of the VCF Operations collector nodes in VCF Operations inventory. | VCF Operations must have connection to the Internet to view and set the precise geographic coordinates in the UI. |
| VCF-OPS-RECD-COM-004 | Activate the Ping integration in VCF Operations. | Provides metrics on the availability of endpoints. | You must activate the integration manually. |
| VCF-OPS-RECD-COM-005 | Configure the currency in the VCF Operations global options based on your organization requirements. | To receive benefits of the VCF Operations cost engine, you must select a currency. | None. |

VCF Operations - Networking and Internet Access Design Choices

There are several options regarding connectivity to the external resources and network placement for the VCF Operations components. You should choose one option from each of the tables below.

Table 3-47. Network Placement for VCF Operations Design Recommendations Default Distributed Port Group

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-NET-001 | Deploy the VCF Operations nodes on the distributed port group created by VCF Installer and reserved for management components. | Ensures that the VCF Operations nodes are deployed adjacent to the other components on the same Layer-2 network. | None. |

Table 3-48. Network Placement for VCF Operations Design Recommendations Dedicated Distributed Port Group

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-NET-001 | Deploy the VCF Operations nodes on a dedicated distributed port group other than default port group reserved for management components by VCF Installer. | Ensures that the VCF Operations nodes are deployed in a dedicated Layer-2 network. | - You must skip the deployment of the VCF Operation (and VCF Operations fleet management) when using the VMware Cloud Foundation Installer. - You must perform configuration to support the networking configuration as a post-deployment task before manually deploying the VCF Operations and its prerequisites. - You must perform deployment using the SDDC Manager API with JSON file as input. - You must ensure that VLAN is provisioned in each availability zone if stretched clusters are used. |

Table 3-49. Network Placement for VCF Operations Design Recommendations VLAN-backed NSX Segment

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-NET-001 | Deploy the VCF Operations nodes on a VLAN-backed NSX segment. | Ensures that the VCF Operations nodes are deployed in a dedicated Layer-2 network. | - You must skip the deployment of the VCF Operations (and VCF Operations fleet management) when using the VCF Installer. - You must perform configuration to support the networking configuration as a post-deployment task before manually deploying the VCF Operations and its prerequisites. - You must perform deployment using the SDDC Manager API with JSON file as input. - You must ensure that VLAN is provisioned in each availability zone if stretched clusters are used. |

Table 3-50. Network Placement for VCF Operations Design Recommendations Ovelay-backed NSX Segment

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-NET-001 | Deploy the VCF Operations nodes on an overlay NSX segment. | Allow portability of the VCF Operations nodes to another VCF Instance in the VCF fleet without the need for reconfiguration of the IP addressing. | - You must skip the deployment of the VCF Operations (aVCF Operations fleet management) when using the VCF Installer. - You must perform configuration to support the networking configuration as a post-deployment task before manually deploying the VCF Operations and its prerequisites. - You must perform deployment using the SDDC Manager API with JSON file as input. |

Table 3-51. Licensing for VCF Operations Design Recommendations Connected Mode

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-LIC-001 | Use VCF Operations in "Connected licensing mode". | - Provides easy license management and up to date states in overall licensing portal. - No manual interaction required. | VCF Operations will require outbound connection to internet directly or through a proxy server. |

Table 3-52. Licensing for VCF Operations Design Recommendations Disconnected Mode

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-LIC-001 | Use VCF Operations in "Disconnected licensing mode". | If your policy does not permit access to internet for management components. | - You need to upload usage statistics from VCF Operations to Broadcom's licensing portal and then download and apply entitlements to all VCF Operations. - Usage statistics has be uploaded to the licensing portal every 180 days. |

Table 3-53. VCF Operations Diagnostics Design Recommendations Manual Upload

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-DIAG-001 | Manually upload Skyline Health Diagnostics and Advisory signatures. | If your policy does not permit access to internet for management components. | Signatures should be downloaded manually and then uploaded into VCF Operations. |

Table 3-54. VCF Operations Diagnostics Design Recommendations Online

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-DIAG-001 | Download Skyline Health Diagnostics and Advisory signatures directly from Broadcom's repository. | Ensures always up to date information about potential issues and mitigation steps in your environment. | VCF Operations will require outbound connection to internet directly or through a proxy server. |

Table 3-55. Binary Management for VCF Operations Design Recommendations Online

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-BIN-001 | Connect to the online depot. | Ensures always up to date information about bundles for Install, Update, Patch and compatibility information. | VCF Operations will require outbound connection to internet directly or through a proxy server. |

Table 3-56. Binary Management for VCF Operations Design Recommendations Offline

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-BIN-001 | Connect to an offline depot. | If your policy does not permit access to internet for management components you may configure an internal depot. | The bundles should be downloaded to a system with access to the Broadcom's depot and then uploaded to the internal depot by following your internal procedures. |

Table 3-57. Binary Management for VCF Operations Design Recommendations Manual

| Design Recommendation ID | Design Recommendation | Justification | Implications |
| --- | --- | --- | --- |
| VCF-OPS-RECD-BIN-001 | Upload files manually. | Removes the overhead of installing and managing an additional appliance for internal depot. | You should download and upload all the files to the respective directories and set the right permissions. A lot of manual labor and error prone process. |

#### SDDC Manager Detailed Design

SDDC Manager provides fleet management capabilities for the underlying virtual infrastructure and uses binaries to deploy new workload domains, and to patch and upgrade existing ones.

Fleet management capabilities provided by the SDDC Manager appliance include the following, the main difference in VMware Cloud Foundation 9.0 is the process for performing these operations is performed with either the VCF Operations UI or vCenter UI.

- Commissioning or decommissioning ESX hosts.

- Deployment of workload domains.

- Adding and extending clusters in workload domains.

- Life cycle management of the virtual infrastructure components in all workload domains.

- Creation of network pools for ESX host configuration workload domains.

- Certificate management of underlying virtual infrastructure.

- Password management and rotation of underlying virtual infrastructure.

- Backup configuration of underlying virtual infrastructure.

SDDC Manager Binary Management

SDDC Manager uses binaries to deploy, patch or upgrade the underlying virtual infrastructure and provides users with flexibility in how they obtain and consume the binaries.

| Mode | Details | Benefits | Implications |
| --- | --- | --- | --- |
| Connected Mode | SDDC Manager maintains a live connection to Broadcom depot via the internet and regularly updates the unified manifest. | Reduced operational overhead. | An outbound internet connection to the Broadcom depot is required for Connected Mode. |
| Disconnected Mode | Offline registration of SDDC Manager with an internal depot. | - Internet connection is not required for SDDC Manager instances. - Internal centrally managed depot. - Increased security posture. | Operational overhead of manual binary download management to populate the offline depot. |
| Manual | Manual download of binaries and upload to an SDDC Manager instance. | Increased security posture. | Operational overhead of manual binary download management. |

Table 3-58. SDDC Manager Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-SDDCM-REQD-CFG-001 | Deploy a new SDDC Manager with each new VCF Instance. | The VCF Installer will automatically install new SDDC Manager with each new management domain. | None. |
| VCF-SDDCM-REQD-CFG-002 | Deploy an SDDC Manager system in the first availability zone of the management domain. | SDDC Manager is required to perform VMware Cloud Foundation capabilities, such as provisioning workload domains, deploying solutions, patching, upgrading, and others. | None. |
| VCF-SDDCM-REQD-CFG-003 | Place the SDDC Manager appliance on the VM management network. | - Simplifies IP addressing for management VMs by using the same VLAN and subnet. - Provides simplified secure access to management VMs in the same VLAN network. | None. |

#### Lifecycle Management Design for VMware Cloud Foundation

In a VMware Cloud Foundation instance, you use SDDC Manager for lifecycle management of the management components in the entire instance except for NSX Global Manager and VMware Aria Suite Lifecycle. VMware Aria Suite Lifecycle manages the lifecycle of the components that it deploys.

Lifecycle management of a VMware Cloud Foundation instance is the process of performing patch updates or upgrades to the underlying management components.

Table 3-59. Lifecycle Management for VMware Cloud Foundation

| Component | Management Domain | VI Workload Domain |
| --- | --- | --- |
| SDDC Manager | SDDC Manager performs its own life cycle management. | Not applicable |
| NSX Local Manager | SDDC Manager uses the NSX upgrade coordinator service in the NSX Local Manager. |  |
| NSX Edges | SDDC Manager uses the NSX upgrade coordinator service in NSX Manager. |  |
| NSX Global Manager | You manually use the NSX upgrade coordinator service in the NSX Global Manager. |  |
| vCenter Server | You use SDDC Manager for life cycle management of all vCenter Server instances. |  |
| ESX | - SDDC Manager uses vSphere Lifecycle Manager baselines or vSphere Lifecycle Manager images to update and upgrade the ESX hosts. - Custom vendor ISOs are supported and might be required according to the ESX hardware in use. |  |
| VMware Aria Suite Lifecycle | VMware Aria Suite Lifecycle performs its own life cycle management. | Not applicable |

VMware Cloud Foundation Lifecycle Management Requirements

Consider the design requirements for automated and centralized lifecycle management in the context of the entire VMware Cloud Foundation environment.

Table 3-60. Lifecycle Management Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-OPS-REQD-LCM-001 | Use VCF Operations to perform the life cycle management of the management components. | Because the deployment scope of VCF Operations covers the full VMware Cloud Foundation platform, VCF Operationsperforms patching, update, or upgrade of these components across all workload domains. | The operations team must understand and be aware of the impact of a patch, update, or upgrade operation by using VCF Operations. |
| VCF-OPS-REQD-LCM-002 | Use vSphere Lifecycle Manager images to manage the life cycle of vSphere clusters. | - With vSphere Lifecycle Manager images, firmware updates are carried out through firmware and driver add-ons, which you add to the image you use to manage a cluster. - You can check the hardware compatibility of the hosts in a cluster against the Broadcom Compatibility Guide. - You can validate a vSphere Lifecycle Manager image to check if it applies to all hosts in the cluster. You can also perform a remediation pre-check. | - Updating the firmware with images requires an OEM-provided hardware support manager plug-in, which integrates with vSphere Lifecycle Manager. - An updated vSAN Hardware Compatibility List (vSAN HCL) is required during bring-up. |

Table 3-61. Lifecycle Management Design Requirements for NSX Federation

| Design Requirement ID | Design Requirement | Justification | Implication |
| --- | --- | --- | --- |
| VCF-OPS-REQD-LCM-003 | Use the upgrade coordinator in NSX to perform life cycle management on the NSX Global Manager appliances. | The version of SDDC Manager in this design is not currently capable of life cycle operations (patching, update, or upgrade) for NSX Global Manager. | - You must explicitly plan upgrades of the NSX Global Manager nodes. An upgrade of the NSX Global Manager nodes might require a cascading upgrade of the NSX Local Manager nodes and underlying SDDC Manager infrastructure before upgrading the NSX Global Manager nodes. - You must always align the version of the NSX Global Manager nodes with the rest of the SDDC stack in VMware Cloud Foundation. |
| VCF-OPS-REQD-LCM-004 | Establish an operations practice to ensure that prior to the upgrade of any workload domain, the impact of any version upgrades is evaluated in relation to the need to upgrade NSX Global Manager. | - The versions of NSX Global Manager and NSX Local Manager nodes must be compatible with each other. - Because SDDC Manager does not provide life cycle operations (patching, update, or upgrade) for the NSX Global Manager nodes, upgrade to an unsupported version cannot be prevented. | The administrator must establish and follow an operations practice by using a runbook or automated process to ensure a fully supported and compliant bill of materials prior to any upgrade operation. |
| VCF-OPS-REQD-LCM-005 | Establish an operations practice to ensure that prior to the upgrade of the NSX Global Manager, the impact of any version change is evaluated against the existing NSX Local Manager nodes and workload domains. | - The versions of NSX Global Manager and NSX Local Manager nodes must be compatible with each other. - Because SDDC Manager does not provide life cycle operations (patching, update, or upgrade) for the NSX Global Manager nodes, upgrade to an unsupported version cannot be prevented. | The administrator must establish and follow an operations practice by using a runbook or automated process to ensure a fully supported and compliant bill of materials prior to any upgrade operation. |

### Information Security and VCF Identity Broker Design

This section describes the design for the Information Security and VCF Identity Broker

#### Access Management Design

Each component in a VMware Cloud Foundation platform has specific access methods. From a security perspective, you must be aware of these access methods to ensure you only grant access to each component to the right users.

Table 3-62. Access Methods for each VCF Component

| Component | Access Method | Additional Information |
| --- | --- | --- |
| SDDC Manager | - UI - API - SSH | SSH is active by default. root user access is not permitted. |
| NSX Local Manager | - UI - API - SSH | SSH is deactivated by default. |
| NSX Edge | - API - SSH | SSH is deactivated by default. |
| NSX Global Manager | - UI - API - SSH | SSH setting is defined during deployment. |
| vCenter | - UI - API - SSH - VAMI | SSH is active by default. |
| ESX | - Direct Console User Interface (DCUI) - ESX Shell - SSH - VMware Host Client | SSH and ESX shell are deactivated by default. |
| VCF Operations | - UI - API - SSH | SSH is active by default. |
| VCF Operations fleet management | - API - SSH | SSH is active by default. |
| VCF Operations collector | - API - SSH | SSH is active by default. |
| VCF Operations for logs | - UI - API - SSH | SSH is active by default. |
| VCF Operations for networks | - UI - API - SSH | SSH is active by default. root user access is not permitted. |
| VCF Automation | - UI - API - SSH | SSH is active by default. |
| VCF Identity Broker | - API - SSH | SSH is active by default. |
| vSphere Supervisor | - UI - API - CLI - SSH | - UI is via vCenter - CLI requires Kubernetes CLI Tools - SSH is active by default. |

#### Account Management Design

You design account management for your VMware Cloud Foundation platform according to industry standards and the requirements of your organization.

Password Management Methods

Multiple methods for managing password life cycle are supported.

Table 3-63. Password Management Methods in VMware Cloud Foundation

| Method | Description |
| --- | --- |
| Update | Update password for a single account with a manually entered password |
| Remediate | Reconcile a single account with a password that has been set manually at the component. |
| Manual | Update a password manually directly in the component. |

Account and Password Management

VMware Cloud Foundation comprises multiple types of accounts: OS accounts, application accounts, and service accounts. Each account has different attributes and is managed according to the specific methods outlined in the table.

For more information on password complexity, account lockout or integration with additional Identity Providers, refer to the Identity and Access Management for VMware Cloud Foundation.

Table 3-64. Account and Password Management in VMware Cloud Foundation

| Component | User Account | Password Management | Additional Information |
| --- | --- | --- | --- |
| SDDC Manager/VCF Installer | admin@local | - Manual by using the SDDC Manager API. - Default Expiry: Never | - Local appliance account.API access (break-glass account). |
|  | vcf | - Manual by using the OS. - Default Expiry: 365 days | - Local appliance account. - OS level access. |
|  | root | - Manual by using the OS. - Default Expiry: 365 days | - Local appliance account. - OS level access. |
|  | backup | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 365 days | - Local appliance account. - OS level access. |
|  | administrator@vsphere.local | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - vCenter Single Sign-On account. - Application and API access. - Additional VMware Cloud FoundationAdmin account required to perform manual password rotation. |
| NSX Local Manager | admin | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account.OS level, API, and application access. |
|  | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
|  | audit | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. - Read-only application level access. |
|  | svc-sddc-manager-hostname-nsx-vip-hostname-uid | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: 90 | Service account between SDDC Manager and NSX. |
|  | svc-ops-nsx-uid | - System managed. | Service account between VCF Operations and NSX. |
|  | svc-ni-nsx-uid | - System managed. | Service account between VCF Operations for networks and NSX. |
|  | svc-vcfa_uid | - System managed. - Default Expiry: Never | Service account between VCF Automation and NSX. |
| NSX Edge | admin | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level, API, and application access. |
|  | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
|  | audit | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. - Read-only application level access. |
| NSX Global Manager | admin | - Manual by using the NSX Global Manager UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level, API, and application access. |
|  | root | - Manual by using each NSX Global Manager appliance. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
|  | audit | - Manual by using the NSX Global Manager UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. - Read-only application level access. |
| vCenter | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. - VAMI access. |
|  | administrator@isolatedsso.local | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - vCenter Single Sign-On account. - Application and API access. - Relevant to isolated workload domain. |
|  | svc-sddc-manager-hostname-vcenter-hostname@vsphere.local | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: 90 | - Service account between SDDC Manager and vCenter. - Administrator role. |
|  | svc-nsx-manager-hostname-vcenter-hostname@vsphere.local | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: 90 | - Service account between NSX Manager and vCenter. - Administrator role. |
|  | svc-vcf-ops-fleet-management-hostname-vcenter-server-hostname@vsphere.local | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: None | - Service account between VCF Operations fleet management and vCenter. - Administrator role. |
|  | svc-ops-vc-vcf-ops-cluster-uid@vsphere.local | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: None | - Service account between VCF Operations and vCenter. - Administrator role. |
|  | svc-vcfa_uid@vsphere.local | - System managed - Automatically rotated every 30 days by default. - Default Expiry: None | - Service account between VCF Automation and vCenter. - Administrator role. |
|  | svc-vcfsp-vc-uid@vsphere.local | - System managed - Automatically rotated every 30 days by default. - Default Expiry: None | - Service account between VCF Services Platform and vCenter. - VCF Services Platform role. |
|  | svc-vcfsp-vc-admin-uid@vsphere.local | - System managed. - Automatically rotated every 30 days by default. - Default Expiry: None | - Service account between VCF Services Platform and vCenter. - VCF Services Platform Admin role. |
| ESX | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | Local root account. |
|  | svc-vcf-ESX-hostname | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | Service account between SDDC Manager and the ESX host. |
| VCF Operations | admin | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: Never | API and application access. |
|  | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
| VCF Operations Fleet Management Appliance | admin@local | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: Never | API and application access. |
|  | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
| VCF Operations Collectors | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
| VCF Automation | vmware-system-user | Update or remediate by using the VCF Operations UI or API. | - Local appliance account. - OS level access. |
|  | admin | - Update by using the VCF Operations UI or API. - Default Expiry: never | Admin account for the VCF Automation system org. |
| VCF Identity Broker | vmware-system-user | Update or remediate by using the VCF Operations UI or API. | - Local appliance account. - OS level access. |
| VCF Operations for logs | admin | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: Never | API and application access. |
|  | root | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 90 days | - Local appliance account. - OS level access. |
| VCF Operations for networks | admin@local | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: Never | API and application access. |
|  | root | - Manual by using the OS. - Default Expiry: 99999 (never) | - Local appliance account. - OS level access. |
|  | support | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | - Local appliance account. - OS level access. |
|  | consoleuser | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | - Local appliance account. - cli access. |
| VCF Operations for networks collector | support | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | - Local appliance account. - OS level access. |
|  | consoleuser | - Update or remediate by using the VCF Operations UI or API. - Default Expiry: 99999 (never) | - Local appliance account. - cli access. |
| vSphere Supervisor | vmware-system-user | Encoded in control plane namespace secret. | - Local appliance account. - OS level access. |

Account Management Design Recommendations

In your account management design, you can apply certain best practices.

Table 3-65. Account and Password Management Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-SEC-RCMD-ACT-001 | Enable scheduled password rotation in VCF Operations for all accounts supporting scheduled rotation. | - Increases the security posture of your VCF fleet. - Simplifies password management across your VCF fleet management components. | You must retrieve new passwords by using the API if you must use accounts interactively. |
| VCF-SEC-RCMD-ACT-002 | Establish operational practice to rotate passwords using VCF Operations on components that do not support scheduled rotation in VCF Operations. | Rotates passwords and automatically remediates VCF Operations databases for those user accounts. | None. |
| VCF-SEC-RCMD-ACT-003 | Establish operational practice to manually rotate passwords on components that cannot be rotated by VCF Operations. | Maintains password policies across components not handled by VCF Operations password management. | None. |

#### Certificate Management Design

You design certificate management for VMware Cloud Foundation according to industry standards and the requirements of your organization.

Access to all management component interfaces must be over a Secure Socket Layer (SSL) connection. During deployment, each component is assigned a certificate from a default signing CA. Replace the default certificate with a trusted enterprise CA-signed certificate to provide secure access to each component.

Table 3-66. Certificate Management in VMware Cloud Foundation

| Component | Default Signing CA | Life cycle for CA-Signed Certificates |
| --- | --- | --- |
| SDDC Manager | Management domain VMCA | VCF Operations |
| NSX Local Manager | Management domain VMCA | VCF Operations |
| NSX Edges | Not applicable | Not applicable |
| NSX Global Manager | Self Signed | Manual |
| vCenter | Local workload domain VMCA | VCF Operations |
| ESX | Local workload domain VMCA | VCF Operations |
| VCF Operations | VCF Operations fleet management locker | VCF Operations |
| VCF Operations fleet management appliance | VCF Operations fleet management locker | VCF Operations |
| VCF Automation | VCF Operations fleet management locker | VCF Operations |
| VCF Operations logs | VCF Operations fleet management locker | VCF Operations |
| VCF Operations networks | VCF Operations fleet management locker | VCF Operations |
| vSphere Supervisor | Local workload domain VMCA | Automatic / Manual |

Table 3-67. Certificate Management Design Recommendations

| Design Recommendation ID | Design Recommendation | Justification | Implication |
| --- | --- | --- | --- |
| VCF-SEC-RCMD-CERT-001 | Replace the default VMCA or signed certificates on all management components with a certificate that is signed by an internal certificate authority. | Ensures that the communication to all management components is secure. | Replacing the default certificates with trusted CA-signed certificates from a certificate authority might increase the deployment preparation time because you must generate and submit certificate requests. |
| VCF-SEC-RCMD-CERT-002 | Use a SHA-2 algorithm or higher for signed certificates. | The SHA-1 algorithm is considered less secure and has been deprecated. | Not all certificate authorities support SHA-2 or higher. |
| VCF-SEC-RCMD-CERT-003 | Perform SSL certificate life cycle management for all management appliances by using VCF Operations. | VCF Operations supports automated SSL certificate lifecycle management rather than requiring a series of manual steps. | Certificate management for NSX Global Manager instances must be done manually. |

#### Embedded VCF Identity Broker Model

The Embedded VCF Identity Broker Model is the simplest form of configuring VCF Single Sign-On for VMware Cloud Foundation. The deployment fits new VMware Cloud Foundation platforms, or deployments that have lower requirements for availability.

Figure 3-10. Embedded VCF Identity Broker Model

Embedded VCF Identity Broker Attributes

The Embedded VCF Identity Broker Model has the following attributes.

| Attribute | Detail |
| --- | --- |
| Service Location | VCF Identity Broker runs as a service within the management domain vCenter. |
| Deployment mechanism | As an embedded service running on vCenter, no additional deployment is required. |
| Number of Nodes | 1 (As part of vCenter) |
| Deployment Timing | As an embedded service running on vCenter, no additional deployment is required. |
| Scale considerations | One VCF Identity Broker per VCF instance. |
| Supported services | For support directories and IdPs, see Identity Providers and Protocols Supported for Single Sign-On. |
| Backup and restore | As part of the vCenter Backup/Restore. |
| High Availability | High Availability is provided by vSphere HA as this service runs as part of the vCenter appliance. |
| Operational Impacts | As this services run as part of vCenter, maintenance tasks performed on the vCenter instance also impact your ability to authenticate to the VMware Cloud Foundation components. |
| Migration path | To migrate from embedded to the appliance it requires a reset of the VCF Single Sign-On configuration. Users must be re-added to components. |

Embedded VCF Identity Broker Model Design Requirements

Table 3-68. Embedded VCF Identity Broker Model Design Requirements

| Design Requirement ID | Design Requirement | Justification | Implications |
| --- | --- | --- | --- |
| VCF-SSO-REQD-EMB-001 | Leverage the embedded VCF Identity Broker in the first management domain vCenter. | - Simplifies the authentication configuration for the VMware Cloud Foundation platform. - Simplifies the operational overhead. | - Limits the Scale supported by the embedded model. - Limits the availability of the authentication service in a VMware Cloud Foundation platform. |
| VCF-SSO-REQD-EMB-002 | Connect the embedded VCF Identity Broker to your corporate IdP. | Provides directory service (users and group) and user authentication service to the VCF Identity Broker service. | None. |

# References

For more information, see the following supplemental configuration and administration guides, white papers and best practices documents.

VMware Pro Cloud Services for Private Cloud Solution References

Remember	Refine or add any additional links to product documentation

See the VMware Cloud Foundation Documentation (https://techdocs.broadcom.com/us/en/vmware-cis/vcf/vcf-9-0-and-later/9-0.html) for product documentation for Cloud Foundation.

The information is arranged by topics that include the following:

- VMware Cloud Foundation 9.0 Release Notes

- VMware Cloud Foundation Planning and Preparation Workbook

- VMware Cloud Foundation Design Guide

- VMware Cloud Foundation Deployment, Convergence and Upgrade Guide

- VMware Cloud Foundation Building Cloud Infrastructure Guide

- VMware Cloud Foundation vSphere Guide

- VMware Cloud Foundation vSAN Guide

- VMware Cloud Foundation NSX Guide

- VMware Cloud Foundation Lifecycle Management Guide

- VMware Cloud Foundation Fleet Management Guide

- VMware Cloud Foundation Infrastructure Operations Guide

- VMware Cloud Foundation Workload Monitoring and Observability Guide

- VMware Cloud Foundation vSphere Supervisor Platform Guide

- VMware Cloud Foundation Security and Compliance Guide

- VMware Cloud Foundation API Reference Guide

VMware Validated Solutions Guides:

- <Add as appropriate>